APPH 226 – Apparatus and Image Management

Worksheet - Size Distortion Formulas and Calculations

Magnification or Size Distortion can be assessed by calculating the magnification factor.

MF =   SID   	***  SOD = SID – OID

SOD

Example:

If the SID is 40 inches, and the SOD is 30 inches, what is the magnification factor?

40 inches    = 1.33

30 inches

The magnification factor is 1.33

The magnification will be 33% or the image will be 133% of the object size.

- What is the magnification factor if the SID is 40 inches and the OID is 6 inches?

- What is the magnification factor if the SOD is 72 cm and the OID is 6 cm?

- What is the magnification factor if the SID is 72 inches and the SOD is 68 inches?

- What is the magnification factor is the OID is 5 cm and the SID is 115 cm?

- What is the magnification factor if the SOD is 48 inches and the OID is 2 inches?

- What is the magnification factor if the SOD is 85 cm and the SID is 100 cm?

The magnification factor permits calculation of the actual size of an image using this formula:

Object size =     	  Image

Magnification Factor

Example:

If an object is 2 inches in diameter on the image, and is measured to be 3 inches away from the IR, what is its actual size if the SID used is 40 inches?

This is the information given:    Image size = 2 inches     SID = 40 inches        OID = 3 inches Step 1.  Solve for the magnification factor first.

M = SID  	  =    40    = 1.08

SOD   	 37 Step 2.  Solve for the object size

O =   2 inches      = 1.85 inches

1.08 	The object size is 1.85 inches in diameter.

  - If the image size is 24 inches, and there is a magnification factor of 2, what is the size of the object that was x-rayed?

  - If a phalange having an area of 2 inches is x-rayed with a 4 inch OID using a 40 inch SID, what would be the size of the phalange on the image?

  - If the object is 13 cm and the image size is 15cm, what is the magnification factor?

  - If the SID is 50 inches, the object is 2 inches and the OID is 2 inches, what is the image size?

Example:

The image of a calcaneus measures 6 cm across, and the actual size of the calcaneus is 5 cm.

What is the percent magnification of the object?

6cm-5cm   x 100 =  20 % magnification of the object

5cm

  - What is the percent magnification if the object is 11 inches long, and the image is 14 inches long?

  - If the object measures 12 inches and shows up on the image as 24 inches, what is the magnification factor?

  - What is the percent of magnification in Question 2 (above question)?

Practice Questions

- Image size = 13 cm   Magnification factor (MF) = 1.7 Calculate the object size.

- Image size = 8 inches    Object size = 5 inches

Calculate the magnification factor.

- SID = 40 inches     Image size = 9 inches    Object size = 7.5 inches Calculate the SOD

- Object size = 13 cm      Magnification Factor (MF)= 1.19

Calculate the image size.

- A mass measures 1.2 cm on an image.  The OID is 8 cm, and the SID is 110 cm.  What is the actual size of the mass?

- SID = 50 inches         Object size = 3 inches   OID = 4 inches Calculate the image size.

- Image size= 26 cm  Object size = 21 cm Calculate the percent magnification

- Image size = 16 cm      Magnification Factor (MF) = 1.8

Calculate the percent magnification

- Image size = 26 cm   Magnification factor (MF) = 1.3

Calculate the object size.

- A gallbladder stone measures 2 cm on the radiograph.  The SID is 100cm and the SOD is 90 cm.  What is the actual size of the stone?