MRAD 302- Specialized Imaging 2

Study Guide Comparison Table Worksheet – ANSWER KEY

| CHARACTERISTICS | ULTRASOUND | MRI | SPECT CT |
| --- | --- | --- | --- |
| Energy Used for the acquisition of information -Is there ionizing radiation present? | -Sound waves 120MHz range or greater -no ionizing radiation | -Magnetic field and (pulsed) radio waves -No ionizing radiation | -radioactive compounds emit positrons during radioactive decay process |
| ALARA- does it apply and if yes in what way, what effects could there be on the body? | -Yes it applies -(TI) Thermal changes are possible which is related to heating and other possibilities are (MI)mechanical changes in cell structure | -Yes it applies -Thermal changes and tissue heating needs to be considered especially in a pregnant woman -Specific Absorption Ratio (SAR)- fetal heating | -Yes radioactive material utilized -CT is used in coordination of scans |
| Is there any specific patient prep needed? | -Full bladder for pelvic exams -NPO for some abdominal exams | -All metal must be removed -Thorough screening form needs to be completed -Orbital x-rays for FB -creatine needs to be determined if contrast is needed (check kidney function) | -NPO for some exams up to 6 hours before May have to follow special diet 12-24 hours before exam Special prep may be needed if diabetic |
| Are there any contraindications for this modality? | -No | -Yes -Pacemakers, cochlear implants, metallic fragments in eye, old style aneurysm clips, internal pain pumps, bone growth stimulators, some surgical clips, pregnancy | -Pregnancy or breast feeding - |
| What are limitations of this modality? | -Limited ability to see air and bone | -Bone does not produce an MRI signal | -Exam and scan times are long |
| What is a gold standard of this modality? | -Gold standard for liver, kidneys, gallbladder, uterus and blood vessels | -Superior soft tissue contrast resolution compared to CT -Better than CT for base of the brain and spinal cord visualization | -Brain mapping, CNS system tumours imaging, and evaluation of dementia |