APPH 226 – Apparatus and Image Management

Lab 8 - Technique Charts

Purpose

Objective: 11.3 Discuss the steps required to create a technique chart.

Assess and modify technical factors.

Instructions - Part One

| Part | Position | BONTRAGER |  | APR |  |
| --- | --- | --- | --- | --- | --- |
|  |  | kVp | mAs | kVp | mAs |
| Forearm | AP |  |  |  |  |
|  | LAT |  |  |  |  |
| Elbow | AP |  |  |  |  |
|  | LAT |  |  |  |  |
| Humerus | AP |  |  |  |  |
|  | LAT |  |  |  |  |
| Facial Bones | Caldwell |  |  |  |  |
|  | Waters |  |  |  |  |
|  | Lateral |  |  |  |  |
|  | Townes |  |  |  |  |
| Ankle | AP |  |  |  |  |
|  | OBL |  |  |  |  |
|  | LAT |  |  |  |  |
| Wrist | PA |  |  |  |  |
|  | OBL |  |  |  |  |
|  | LAT |  |  |  |  |
| Tib/Fib | AP |  |  |  |  |
|  | LAT |  |  |  |  |
| Sacrum | AP |  |  |  |  |
|  | LAT |  |  |  |  |

- Using the APR on the console, and a textbook, complete the technique chart provided.

Questions:

- Are there any differences between the techniques given on the APR of the room compared to the techniques provided by Bontrager? If yes, why?

- Which techniques would you follow?

Instructions  - Part Two

- Select a phantom, and take an image using the exposure factors from the technique chart you completed. *Remember to position the phantom appropriately to match the technique chart specifications. Use the textbook as a resource*

- Assess the image, and record the EI onto the chart below.

- Make adjustments as needed to the technical factors in order to achieve an exposure in the ideal EI range. For the purposes of this lab, aim for the exact middle of the acceptable EI range.

- Record the ideal factors into the chart below.

| Part | Position | kVp | mAs | EI | Acceptable? | New kVp | New mAs | EI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Yes/No |  |  |  |
|  |  |  |  |  | Yes/No |  |  |  |
|  |  |  |  |  | Yes/No |  |  |  |
|  |  |  |  |  | Yes/No |  |  |  |
|  |  |  |  |  | Yes/No |  |  |  |
|  |  |  |  |  | Yes/No |  |  |  |
|  |  |  |  |  | Yes/No |  |  |  |

- Using your knowledge about how to manipulate mAs, create a technique chart for different sized patients based on the “ideal” technical factors that you tested and recorded in step 5. Make the adjustments based on the theory from the textbook.

| Part | Position | Average Technique | Small | Large |
| --- | --- | --- | --- | --- |

Questions:

- What is the advantage of keeping the kVp constant when designing a technique chart?

- How should the technique be adjusted for a hypersthenic patient?

- If an osteoporotic patient came into the department, should the technologist use a technique chart for them?

- List and explain 3 situations where a technologist would utilize a technique chart instead of using AEC.

- Using information for one of the x-ray rooms, complete the following technique chart with the new technical factors to achieve optimal exposures

Ideal Range for EI: 1800-2100

Relationship: Doubling mAs will increase EI by 300

| Part | Position | kVp | mAs | EI | Acceptable? | New kVp | New mAs | EI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hand | PA | 52 | 2 | 1800 | Yes/No |  |  |  |
| Humerus | Lateral | 60 | 1.6 | 1700 | Yes/No |  |  |  |
| Shoulder | Lateral | 81 | 64 | 2300 | Yes/No |  |  |  |
| Foot | AP | 57 | 2.5 | 1910 | Yes/No |  |  |  |
| Knee | Obl | 66 | 3 | 1732 | Yes/No |  |  |  |
| Hip | AP | 77 | 8 | 1605 | Yes/No |  |  |  |
| Pelvis | AP | 77 | 10 | 1550 | Yes/No |  |  |  |