RADP 215 – Radiation Protection

# Activity: Entrance Skin Exposure/Entrance Skin Dose Calculations

# Questions 1 - 11

## Purpose:

To apply the correct formulas to calculate entrance skin exposure/entrance skin dose calculations.

## Instructions:

- Refer to the following chart to answer the questions below.

- Upload a completed copy into EDENBROOK.

- Your instructor will review the answers with the class.

| mGy/mAs @ 110 cm SID |  |
| --- | --- |
| kVp | mGy/mAs |
| 50 | 0.01 |
| 60 | 0.02 |
| 70 | 0.025 |
| 80 | 0.03 |
| 90 | 0.035 |
| 100 | 0.04 |
| 110 | 0.045 |
| 120 | 0.05 |

- At 70kVp what is the output according to the chart?

- At 120 kVp what is the output according to the chart?

- How does output change as the kVp increases?

- What is the intensity using an output of 60 kVp @ 2 mAs?

- What is the intensity using an output of 100 kVp @ 100 mAs?

- What is the formula for the inverse square law?

- Using the table above, what is the approximate entrance skin exposure for a 15 cm AP hip produced at 80 kVp @ 20 mAs? SID is 110cm and patient distance from IR is 8cm.

- Using the table provided, what is the approximate ESE for a 30 cm lateral L-spine produced at 90 kVp @ 64 mAs? SID is 110 cm and patient distance from IR is 8cm.

- Using the table provided, what is the approximate ESE for a 25cm PA chest produced at 110 kVp @ 2.0mAs and an SID of 180 cm is patient distance from IR is 8cm.

Using the nomogram on pg. 570 in Bushong textbook answer the following questions:

- Estimate the ESE from an AP shoulder x-ray using 100cm. The technical factors used were 80 kVp @ 12.5mAs. The unit has 3.0 mm filtration

- Estimate the ESE from a PA hand x-ray using 100cm SID. The technical factors used were 60kVp @ 2mAs. The unit has 2.5 mm filtration