APPH 226 – ANSWER KEY

The Effect of Variables on Photographic Properties of a Radiographic Image

| Variable | Radiographic Density | Reason | Radiographic Contrast | Reason |
| --- | --- | --- | --- | --- |
| ↑ mAs | ↑ | more photons in beam | N/A* | Only if resulted in overexposure |
| ↓ mAs | ↓ | less photons in beam | N/A* | Only if resulted in underexposure |
| ↑ kVp | ↑ | more photon transmission | ↓ | more scatter, more penetrability of beam, more uniform density |
| ↓ kVp | ↓ | less photon transmission | ↑ | Less scatter, more differential absorption |
| ↑ Atomic Number of target | ↑ | higher efficiency xray production, higher energy xrays | ↓ | more scatter, more penetrability of beam, more uniform density |
| ↓ Atomic Number of target | ↓ | Less efficiency of xray production, lower energy xrays | ↑ | less scatter, more differential absorption |
| ↑ OID | ↓ | less scatter | ↑ | Less scatter |
| ↓ OID | ↑ | more scatter | ↓ | More scatter |
| ↑ Grid Ratio | ↓ | less scatter, some of primary absorbed | ↑ | Less scatter |
| ↓ Grid Ratio | ↑ | more scatter | ↓ | More scatter |
| ↑ Collimation | ↓ | less scatter | ↑ | Less scatter |
| ↓ Collimation | ↑ | more scatter | ↓ | More scatter |

| ↑ Atomic Number of part | ↓ | more absorption occurs | ↑ | Less scatter |
| --- | --- | --- | --- | --- |
| ↓ Atomic Number of part | ↑ | less absorption occurs | ↓ | More scatter |
| ↑Part Thickness | ↓ | more absorption occurs (exponential attenuation) | ↓ | More scatter |
| ↓Part Thickness | ↑ | less absorption occurs | ↑ | Less scatter |
| ↑ Inherent Filtration | ↓ | photons filtered out of beam | ↓ | more scatter, more penetrability of beam, more uniform density |
| ↓ Inherent Filtration | ↑ | more photons in beam | ↑ | Less scatter, more differential absorption |
| ↑ SID | ↓ | fewer photons in beam due to divergence | N/A* | Only if resulted in underexposure |
| ↓ SID | ↑ | more photons in beam, less distance to diverge | N/A* | Only if resulted in overexposure |
| ↑ Field size | ↑ | more scatter | ↓ | More scatter |
| ↓ Field size | ↓ | less scatter | ↑ | Less scatter |
| ↑ exposure time | ↑ | more photons in beam | N/A* | Only if resulted in overexposure |
| ↓ exposure time | ↓ | less photons in beam | N/A* | Only if resulted in underexposure |