RADP 215 – Radiation Protection

ESE/ESD Calculations Worksheet – ANSWER KEY

Complete the following chart:

| SID (cm) | IR to TT (cm) | Body part (cm) | SSD (cm) |
| --- | --- | --- | --- |
| 100 | 5 | 5 | 90 |
| 130 | 7 | 20 | 103 |
| 150 | 6 | 14 | 130 |
| 180 | 8 | 23 | 149 |
| 100 | 0 | 10 | 90 |

Nomogram

Intensity Chart

| mGy/mAs chart at 110 cm |  |
| --- | --- |
| kVp | mGy/mAs |
| 50 | 0.009 |
| 60 | 0.017 |
| 70 | 0.025 |
| 80 | 0.035 |
| 90 | 0.045 |
| 100 | 0.056 |
| 110 | 0.069 |
| 120 | 0.079 |

Complete the chart using the provided nomogram:

|  | kVp | mAs | Al (mm) | SSD (cm) | Estimated ESE | Nomogram read out used |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | 50 | 10 | 2 | 84 | 0.43 mGy | 3 mGy/100mAs |
| 2. | 70 | 50 | 3 | 112 | 1.59 mGy | 4 mGy/100mAs |
| 3. | 80 | 125 | 3.5 | 132 | 3.23 mGy | 4.5mGy/100mAs |
| 4. | 125 | 2 | 4 | 162 | 0.08 mGy | 10mGy/100mAs |

Complete the following 2 questions using the provided intensity chart:

5.  Calculate the approximate ESE for an AP hip exam.

Body part thickness: 20 cm.

Distance from the patient to the IR: 7 cm.

70 kVp @ 20 mAs

SID: 150cm

ESE = 0.4 mGy

6. Calculate the approximate ESE for a PA chest image 40 cm thorax.

Body part thickness: 40 cm.

Distance from the patient to the IR: 5 cm.

120 kVp @ 5 mAs

SID: 180cm

ESE = 0.26 mGy

Space provided for calculations: