MRAD 281 – Computed Tomography Theory 2

Worksheet - Introduction to CT Scanning

Elements of a CT Protocol

Answer the following review questions to test your knowledge of the following important elements of a CT Protocol Sheet.

- What is a scout image?

- What are some other names for a scout image?

- What projection(s) is a scout image?

- Telebrix or barium is an example of what type of contrast?

- Telebrix or barium used in CT will highlight what body system(s)?

- IV contrast used in CT is typically ionic or non-ionic?  Why?

- IV contrast used in CT will highlight what body system(s)?

- What is DFOV?

- What is SFOV?

- How does changing the DFOV affect image quality?

- Window Width controls image ________________________.

- Window Level controls image _______________________.

- Determine the appropriate upper and lower values for the WW for the following scenarios / settings:

  - WW 300, WL 50

  - WW 150, WL 0

  - WW 1000, WL 250

  - WW 80, WL -20

- We know the CT number for water is 0.  If a scan is performed with window settings of WW 100, WL 100, how will water appear on the images acquired?

- If the same scan is performed with window settings of WW 100, WL 20, how will water appear on the images acquired?

- What does MPR stand for?

- List at least 3 other post-processing techniques / filters that can be applied to CT scans.