# Activity: Protecting Patients – ANSWER KEY

Technical Factors:

Appropriate techniques are essential to achieve a diagnostic image with minimal patient dose

- What are 7 methods that we can use to try & reduce patient dose?

- Technical factors

- Shielding

- Collimation

- Filtration

- Communication

- Equipment (Grids, immobilization devices)

- Quality Control

- Why is it important to have technique chart for each room?

Standardized for each unit, and IR specifications

When not using AEC, consult chart before exposure

Lists body parts, sizes to minimize guessing

- What is the ideal kVp & mAs combination for the least patient dose?

Ideal technique for least dose to the patient would be high kVp & low mAs

Not always practical

Not always ideal image quality

- Why is that combination of factors above the best for lowing the dose?

kVp controls quality of beam (penetrating power)

mAs is the quantity (number of photons)

If the penetrating power is high, more photons will make it through the patient & to the IR

Less absorption by patient, lower patient dose

More photons hit the IR, less mAs is required to maintain density on the image, lower technique, less patient dose

- What changes to image quality may be visualized when an image is produced with a high kVp and low mAs?

Image produced may be of poor quality

Reduction of radiographic contrast

- How is the optimum technique determined?

Balance must be maintained between image quality & dose

Use the highest practical kVp & lowest mAs that will produce an optimal image

Determined by numerous exposures on phantoms

- What are most common SID’s used when creating technique charts & factors

In Canada: 100, 110, 150, & 180 cm & In the USA:  40” or 72”

- What is the outcome if an exposure is taken at a SID that is too short?

Overexposed may require repeat

Unnecessary dose to the patient

- What is the outcome if an exposure is taken at a SID that is too long?

Underexposed may require repeat

Unnecessary dose to the patient

- Name & describe the construction & use the 4 types of patient’s shields commonly used?

- Flat Contact Shields

Made of lead strips or material with lead in it

Can be placed right on patient

Most commonly seen in x-ray departments

- Shadow Shields

Made of a radiopaque material

Hangs over area of interest to cast a shadow over area to be protected

Be careful not to obstruct anatomy

- Shaped Contact Shields

Made of radiopaque material contoured to enclose the male reproductive organs

Like an athletic cup

Maximal gonadal protection in any projection, except for PA

- Clear Lead Shields

Made of transparent material with approximately 30% lead

Usually for scoliosis exams – breast & gonadal shielding

- When do we Shield?

As often as possible!!

When gonads are within about 5 cm of the primary beam

- When would we choose not to use shielding?

If it obstructs the area of interest (female pelvis)

- Why is collimation first & most important step in dose prevention & shielding?

Collimated beam will reduce dose more effectively than shielding

Absorbing the primary photons not required in the field at the collimator

Best practice states the primary beam:

Must not be larger than the size of the IR being used for the exam

Should be collimated to the anatomy of interest

Decreased field size, decreased dose, scatter & improved image contrast

- What is the advantage of “Positive Beam Limitation” (PBL)

Sensors in a cassette holder automatically sends a signal to set the collimator to match the CR cassette size

Detectors field size is set when the exam is selected in the APR

- What are the 3 types of beam limitation devices?

Aperture diaphragm

Cones

Cylinder

- What are the advantages of filtration in regard to dose & imaging?

Reduces exposure to the patient’s skin & superficial tissue

‘Hardens’ the beam

Photons in the beam are more penetrating & less likely to be absorbed

Low energy photons contribute only to dose & not to the image

Different filters for different equipment, depending on purpose

E.g. Mammography- soft tissue imaging

- How would a compensating filter be used?

Used when imaging an anatomy that has large variance in thickness or tissue composition

Helps even out the density of the anatomy giving an even contrast on the image

All anatomy can be clearly visualized on the image, instead of having to take two images

- What is the location & purpose of a grid?

Placed in between patient & image receptor

Removes scattered photons before it reaches the IR

- What are the advantages of a grid in regard to the image?

Significantly improves radiographic contrast & detail

Greater quantity of diagnostic information is available with grid

- What are the disadvantages of a grid in regard to the patient dose?

Increases dose because needs additional technique

Compensation for the loss of the scatter & primary photons

Dose increases as grid ratio increases

Improved image quality greatly outweighs the additional dose

- How can good communication with your patient make an examination more successful & help reduce patient dose?

Reduces anxiety & stress

Enhances your professional image as a health care professional

Increases patient cooperation during the exam

Dose reduction!

- Why is it important to explain the procedures to your patients, especially lengthy examinations?

Unknown is scary

Ensures patients understand what you need & you understand their limitations

Unable to stand, cannot lay on left side, etc

- What should technologist do never do when explaining procedure to a patient?

Never lie to the patient!

If something is going to be uncomfortable

Tell them but don’t over emphasize or dwell on the discomfort

- What is the outcome on the image when there is motion during an exposure?

Results in a blurry image

Little/no diagnostic value

Requires a repeat

- What are the two types of motion that can cause blurring of an image?

Voluntary

Involuntary

- What the purpose of giving the patient breathing instructions?

To eliminate voluntary motion

Minimizing repeat images

- What equipment can be used to effectively immobilize a patient or body part?

Sponges

Tape

Support Stands

Pigg-o-stat

Octostop

Sandbags

- Why is it important to have a quality control program in a facility?

Ensures production of optimal quality images

Equipment function & monitoring for peak efficiency

Monitors procedures for performance, monitoring & quality control

- What would happen if a facility is without a quality control program?

Technique that is set may not be what was actually produced

Collimation & centering may be inaccurate

Causing cut-off of anatomy of interest

Non-diagnostic images that need to be repeated

Causing unnecessary dose to the patient

Protection Standards

- Identify the 5 major organizations from the descriptions below responsible for evaluating risks with radiation doses & biologic effects?

- ICRP

Considered the international authority regarding the safe use of ionizing radiation

Evaluates information on biologic effects of radiation

Provides radiation protection guidance through general recommendations on dose limits

- NCRP

US Corporation

Non-government group

Determines how ICRP recommendations are incorporated into US radiation protection criteria

- UNSCEAR

Helps form radiation protection guidelines

Evaluate human & environmental ionizing radiation exposures

Derives risk assessments from epidemiologic data & provides information to organizations (like ICRP) for evaluation

- NAS/NRC – BEIR

Advisory group that reviews studies of biologic effects of ionizing radiation & risk assessment

Also study groups like early radiation workers, atomic bomb victims & nuclear accidents

Provides information to organizations like ICRP for evaluation

- Health Canada

Federal department responsible for helping the Canadian people maintain & improve their health

Responsible for the SC35 Safety Code

- What is the purpose of these groups?

  - Bottom line:

  - These groups help to form recommendations for EfD & EqD limits

  - Limits on radiation exposure are developed based on research & information from these groups

  - Protects the technologist, patient & the general public