APPH 226- Apparatus and Image Management

Worksheet - Distortion and Detail

Questions 1-30

- In your own words, define distortion.

- What are the two types of distortion?

- What aspect of the examination is distortion directly related to?

- To minimize distortion, what factors must be paid careful attention?

- How does increasing magnification affect recorded detail?

- What is the advantage to performing a chest x-ray PA instead of AP?

- Why is a larger SID used for chest x-rays?

- If there is a large OID present, and it cannot be avoided, what can be done to help decrease size distortion?

- Object 1 and Object 2 are the same size.  On an image, Object 2 appears larger than Object 1.

Which object is closer to the image receptor?

- Which of the following changes would decrease magnification?

    - Decrease OID

    - Increase OID

    - Decrease SID

    - Increase SID

- Define shape distortion.

- Foreshortening occurs when:

- Elongation alone occurs when:

  - The tube is angled (CR not perpendicular to IR and part)

  - The IR is angled (not parallel to part)

  - The part being examined is not parallel to the image receptor

- Elongation with magnification occurs when:

  - The tube is angled (CR not perpendicular to IR and part)

  - The IR is angled (not parallel to part)

  - The part being examined is not parallel to the image receptor

- The correct technique to ensure that no shape distortion will occur is to have the part and IR parallel, and the CR perpendicular to both.

  - True

  - False

- Why is it important to have the central ray centered over the joint/area of interest?

- As the part lies further from the IR, shape distortion will decrease.

  - True

  - False

- Would a technologist ever angle the tube on purpose? Why or why not?

- Define recorded detail.

- What unit is resolution measured in?

- What has more detail?

  - An object

  - An image of that object

- The ability of an imaging system to record two adjacent structures as separate structures is called:

  - Definition

  - Recorded detail

  - Resolution

  - Visibility of detail

- The accuracy of structural lines recorded on a radiographic image is called:

  - Recorded detail

  - Resolution

  - Visibility of detail

  - Distortion

- In regards to the resolution test tool, what does a line pair consist of?

- What are five factors that affect recorded detail?

- Which of these factors will increase the detail of an image?

    - Increased focal spot size

    - Decreased focal spot size

    - Increased SID iv.	Decreased SID

    - Increased OID

    - Decreased OID

- Which term applies to the unsharp shadow around an image?

  - Edge gradient

  - Umbra

  - Penumbra

  - Area of partial illumination

- True or False?  A focal spot cannot image anything smaller than its own size.

- Why does a longer SID help to improve recorded detail?

- How does OID affect recorded detail and why?