RADP  215– Mobile Radiography

Readings

| Sherer – Pg 294 | Bontrager Pg 591 | SC 35 Section A – 2.2 Pg 11 |
| --- | --- | --- |

Radiation Safety

- Produces some of the highest occupational radiation exposure for technologists (fluoro gives the most)

- Need to consider protection for:

  - Technologist

  - People in immediate area

  - Patient

- Three Cardinal Principles

  - Time

  - Distance

  - Shielding

Time

- Staff rotation through area – higher occupational exposure

- Mobile exams should be performed ONLY if the condition of the patient prevents them from being able to go to the department

          - SC 35 Section A 2.2

- Is this always the case?

Distance

- SID should be 40 inches for most exams, 72 inches for chest x-rays

- If your SID is too large, it results in needing to use additional mAs to compensate for the extra distance

- Best practice: use that tape measure!

Distance for You

- Must be at least 3 meters from the x-ray tube unless wearing lead or standing behind a leaded shield

  - Says the SC 35 Section A 2.2

  - This is the MINIMUM distance!

  - However… best practices, is to stand as far away from the patient, tube and beam as possible!  Use the whole length of the cord

Distance for Others

- Need to inform all persons in immediate area that an x-ray exposure is about to occur

- Give people time to leave the area

  - YELL LOUD!

  - “X-RAY!!  BED 24!” pause…take a look… then expose…

Where should you stand?

- Whenever you can, you should stand at least 6 feet (2 meters) from the patient and useful beam.

  - Ideally, as far away as possible!

- The lowest amount of scatter radiation occurs at a right angle (90 degrees) from the primary x-ray beam.

  - Shielding and distance have a greater effect on reducing exposure and should always be considered first

Shielding

- Who needs it?

  - You

  - Patient

  - Anyone holding (IR or patient)

  - Surrounding individuals

- You should have an apron on

- The patients gonads should be shielded if possible provided it doesn’t interfere with the exam

- Individuals that cannot leave the area should be shielded

- Anyone holding the patient or IR for the exposure MUST have shielding

What do you do if….

- You arrive on a unit for a portable, the 36 year old female patient needs to be held for the x-ray by a nurse as she is unsteady.  The patient’s roommate is a 30 year old woman.  You have two full aprons and one half apron.