APPH 226 – Apparatus and Image Management

Lab 9 – Differential Absorption

Purpose

Objective: 5.7 Identify patient variables which may impact the image. Understand absorption characteristics in relation to radiographic densities.

Instructions

Exposure #1

- Add an abdomen to the examination.

- Set up for an exposure on the table detector with an SID of 110 cm.

- Ensure AEC is selected.

- Place a Plexiglas block on the table and center it in the light field.

- Collimate to the edges of the Plexiglas block.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure and evaluate the image.

Image Evaluation Questions:

- Describe the image and note any differences in density throughout the image.

- Discuss the absorption characteristics of the object as they relate to the radiographic densities produced.

Exposure #2

- Complete the “Prediction” column of the chart below.

- Add a hand to the examination.

- Select PA hand, and maintain the setup from Exposure #1.

- Take one exposure of each of the following (on top of the Plexiglas block):

  - Glove filled with air

  - Glove filled with water

  - Glove filled with iodine contrast

  - Glove filled with oil

- Place all 4 gloves on top of the Plexiglas block, ensuring no overlap of the gloves.

- Use a small letter marker for each glove to keep track of what is inside of each glove.

- Collimate to include all of the gloves in the light field.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure and evaluate the image.

| Substance | Atomic Number (Z) | Mass Density (kg/m3) | Prediction (darkest - lightest) | Actual (darkest - lightest) |
| --- | --- | --- | --- | --- |
| water | 7.4 | 1000 |  | Medium (2) |
| air | 7.3 | 1.2 |  | Darkest (1) |
| oil | 35 | 9.2 |  | Medium (3) |
| iodine | 53 | 4930 |  | Lightest (4) |

| Soft tissue | 7.4 | 1000 |
| --- | --- | --- |
| Lung | 7.4 | 320 |
| Fat | 6.3 | 910 |
| Bone | 13.8 | 1850 |
| Barium | 56 | 3500 |

Image Evaluation Questions:

- Compare the results from the “Actual” column to your “Predictions” column. Were your predictions correct?

- Why is there a difference in attenuation between the air and the water filled glove?

- Which 2 gloves are visually the most comparable? Explain why they attenuate similarly.

Questions:

- Define differential absorption.

- What are the factors that contribute to differential absorption? Explain the relationship.