Purpose

APPH 226 – Apparatus and Image Management

Lab 4 – SID and Filtration

Objective: 5.2 Discuss the effect photographic properties have on diagnostic quality. Understand the relationship between SID and the amount of radiation that reaches the IR. Instructions

Exposure #1

- Select an elbow or foot phantom.

- Enter the patient information into the console, select table detector, and ensure the SID is 110cm.

- Place the phantom and dosimeter in the light field, and collimate appropriately.

- Set 60kVp @ 3.2mAs (*or 3.1mAs).

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout:

- Record the results from the dosimeter:	mR Exposure #2

- Maintain the same setup as Exposure #1.

- Adjust the SID to 65cm.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout:

- Record the results from the dosimeter:	mR

Exposure #3

- Maintain the same setup as Exposure #1.

- Adjust the SID to 140cm.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout:

- Record the results from the dosimeter:	mR

## Image Evaluation:

- Evaluate the images and note the density of each image. Which image displays the most radiographic density? Which image displays the least amount of radiographic density?

At a glance, they all look very similar. This is because of the digital algorithm applied to each image.

- Which images would you consider to be diagnostic?

They are all diagnostic. Some results may show Exp 3 to be slightly grainy.

- Which image is underexposed or overexposed based on the EI readout?

## Questions:

- How does distance affect radiographic density?

- At a longer SID, how are the photons in the beam affected (at the level of the IR)?

- In order to maintain the same radiographic density in Exposure#1, using a distance of 180cm, what should the mAs be set to?

### Using the density maintenance formula: example below

- What are some examples of situations where a technologist would apply the direct square law?

- What is another name for the direct square law?

- How does the intensity of the radiation change as the SID is decreased?

- If the SID had to be changed, should the technical factors be modified? If yes, would you change the kVp or mAs? Explain your rationale.

# Filtration & Contrast

Objective: 5.4 Manage scatter control and its effect on the image. Determine how filtration affects the beam and image quality.

## Instructions

Exposure #1

- Enter the patient information into the console.

- Select table detector, and use an SID of 110cm.

- Place dosimeter in the middle of the light field and collimate appropriately.

- Set 60kVp @ 12.5mAs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter:	mR. Exposure #2

- Maintain the same setup as Exposure #1.

- Tape 1 mm of Al filtration to the collimator.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter:	mR. Exposure #3

- Maintain the same setup as Exposure #1.

- Tape 2 mm of Al filtration to the collimator.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter:	mR. Exposure #4

- Maintain the same setup as Exposure #1.

- Tape 3 mm of Al filtration to the collimator.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter:	mR.

## Questions:

- Which exposure had the lowest reading? Why do you think that happened?

- If filtration is added, will any adjustments to the technical factors be required?

- What effect does added filtration have on the average energy of the x-ray beam?

- Would additional filtration increase or decrease the dose to a patient?

- Please draw what the emission spectrum would look like if no changes were made except for adding filtration to the beam. Example below.

No filtration

With added filtration