Activity: CT Detectors

## Purpose:

Objective 2.2: Characterize the various acquisition components that comprise a CT scanner.

## Instructions:

- Answer the following true or false questions.

Questions

- Detectors record attenuation information.

  - Fact

  - Fiction

- Detectors convert electrical signals into digital signals.

  - Fact

  - Fiction - convert photon E into electrical signals

- A good detector will have almost no lag.

  - Fact

  - Fiction

- A wide dynamic range means the detector can measure more signals.

  - Fact

  - Fiction

- Solid State Crystal Detectors are used in conventional CT scanners.

  - Fact

  - Fiction - used in modern scanners

- Solid State Crystal Detectors are faster because they use direct conversion.

  - Fact

  - Fiction – 2 step conversion

- Xenon detectors are comprised of separate gas chambers and a photodiode.

  - Fact

  - Fiction - no photodiode required

- Xenon detectors exhibit a lot of afterglow, but are inexpensive.

  - Fact

  - Fiction  - no afterglow

- Xenon detector design prevents them from being used in MDCT.

  - Fact

  - Fiction

- Solid State Crystal Detectors have 50-87% efficiency.

  - Fact

  - Fiction – Xenon detectors do; nearly 100% for Solid-state

- The relative placement, size, and shape of the detectors affect the amount of scatter radiation that reaches the image.

  - Fact

  - Fiction

- A Short wide detector design will absorb less scatter.

  - Fact

  - Fiction - it will absorb more scatter

## Instructions:

Fill in the chart as: Xenon or Solid-State *Review Romans pg. 18 & Table 2-1

| Detector Type | Detector Characteristics |
| --- | --- |
| Xenon | Highly stable |
| Solid-State | Can exhibit afterglow |
| Xenon | Aluminum casing |
| Solid-State | High photon absorption |
| Solid-State | Sensitive to temperature |
| Xenon | Low-density material |
| Xenon | Low QDE |

Instructions:

Fill in the chart as: SDCT or MDCT

| System Type | CT System Characteristics |
| --- | --- |
| MDCT | Enables retrospective slice thickness |
| MDCT | Detectors can be used in various combinations |
| MDCT | Matrix array |
| SDCT | Collimation determines slice thickness |
| SDCT | A complete tube rotation produces a single slice |
| MDCT | Solid-state detector systems |
| MDCT | Adaptive array |
| SDCT | Ionization detectors |

- What controls the slice thickness in a SDCT scanner? Is this different for a MDCT unit? Explain your answer.

SDCT = Collimation

MDCT = Collimation & number of detectors

- How does slice thickness affect spatial resolution?

Larger (aka. thicker) slices = lower (↓) spatial resolution

Thinner slices = higher (↑) spatial resolution

*recall: spatial resolution determines if small objects can be displayed on a CT image