APPH 226 – Apparatus and Image Management

# Lab 10 - Detail and Distortion

### Phantoms Upper and Lower Extremities

### Purpose

Objective: 5.6 Discuss the effect that geometric factors have on diagnostic quality.

Demonstrate how distortion is influenced by alignment.

### Instructions:

*For exposures use 55kVp @ 1.6mAs with a 110cm SID. Collimate to the edges of the anatomy.

Exposure #1

- Position the phantom (upper extremity/lower extremity) lengthwise on the tabletop.

- Center the crosshairs over the middle of the bone.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of one body part on the image: ___________

Exposure #2

- Maintain the position of the phantom.

- Angle the tube 200 cephalad.

- Align the tube and detector and ensure the body part is centered in the crosshairs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of the same body part as in exposure #1 on the image: ___________

Exposure #3

- Tape the body part on a 450 sponge, so that one end is up higher from the tabletop.

- Align the tube (no angle) and detector and center the body part in the crosshairs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of the same body part on the image: ___________

Exposure #4

- Increase OID by stacking several positioning sponges, but ensure the surface is level. Place the boy part on top of the sponges.

- Measure and record the amount of OID used ___________

- Center the body part in the crosshairs.

- Ensure everyone is out of the room & the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure

- Measure and record the size of the same body part on the image: ___________

Results:

Object size ____________ mm

| Image | Image size(mm) | Distortion type and observation |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |

Image Evaluation Discussion:

- Compare images 1-4. How are the different types of distortion visualized? Record observations in the table.

- How can shape distortion caused by improper positioning contribute to size distortion? Is this demonstrated in any of the exposures?

Discussion:

- How does SID affect distortion?

- Name an x-ray procedure in which shape and/or size distortion is used intentionally to our advantage and how it is accomplished.

- In order to limit the effects of size and shape distortion on a radiographic image, what must the technologist always ensure prior to an exposure?