MRAD 358 – Clinical Integration 1

Computed Tomography (CT) Review – Questions 1-23

- What should you communicate to all patients prior to an enhanced scan?

- How long should you monitor a high-risk patient post-scan? Identify the type of reaction that warrants this type of monitoring.

- Why is patient hydration an important consideration?

- How would your image be affected if you had to use Optiray 350 instead of Optiray 300?

- Can you utilize a PICC line to administer IV contrast media? Why or why not?

- The warming device brings the contrast media to body temperature.  True or False?

- Which substance should be loaded into the syringe that has a green display on the control panel and which goes into the syringe with the blue display?

- What can the technologist do to prevent causing an air embolus in the patient?

- A vertical plane that passes through the body, dividing it into anterior and posterior portions, is the _____ plane.

  - sagittal

  - coronal

  - axial

  - oblique

- The plane that passes diagonally between the axes of two other planes is the _____ plane.

  - Sagittal

  - Coronal

  - Axial

  - Oblique

- The dorsal cavity can be further subdivided into which of the following cavities?

- Cranial and spinal

- Thoracic and abdominopelvic

- Two lateral pleural

- Abdominal and pelvic

- Which of the following is a parameter that allows for the adjustment of the gray scale?

- CT number

- Hounsfield unit

- Window width

- Window level

- Water is used as a reference tissue and is given the CT number of:

- –1000

- –500

- 0

- +1000

- Slice Thickness in CT is determined by:

  - Technologist setting

  - Focal spot size

  - Collimator width

  - Radiologist preference

- How does noise appear on a CT image?

  - Decreased contrast

  - Increased brightness

  - Graininess

  - Streak artifact

- Increasing the pitch will result in a scan covering more anatomy lengthwise for a given total acquisition time.  True or false?

- Patients with asthma must be identified prior to injection of contrast media because:

  - Their renal function may be impaired

  - The contrast media may cause hypertension in an asthmatic patient

  - The risk of allergic reaction is higher

  - The radiologist will cancel the exam

- What is the Scan Field of View?

  - The area within the gantry from which raw data is acquired

  - The size of the computer matrix

  - The collimation applied to the images

  - The region of interest selected by the operator

- Which of the following statements is true?

  - The DFOV cannot be larger than the SFOV

  - The DFOV is a subset of all the scan data available

  - Changing the DFOV changes the pixel size

  - Changing the DFOV does not affect image quality

- What is another name for a bone algorithm?

  - Standard

  - Soft-tissue

  - High resolution

  - High contrast

- The image below is an example of which of the following?

  - Multiplanar reformation

  - Shaded surface display

  - Maximum intensity projection

  - Volume rendering

- When would it be beneficial to apply the edge enhancement feature?

- When would it be beneficial to apply the image smoothing feature?