APPH 226 – Apparatus and Image Management

Worksheet - Density- Other Factors

- What is the relationship between dose and distance?

- What does FFD stand for?

- Does the focal spot size influence the density of an image?

- What is the formula for the inverse square law?

- What is the formula for the density maintenance formula?

- What is another name for the density maintenance formula?

- What is the relationship between distance and density maintenance?

- What does OID stand for?

- How can OID affect radiographic density?

- What situations would require the use of the density maintenance formula?

- Meet Slim Jim.

  - What classification would his body habitus fall under?

  - At a glance, would a technologist need to make any changes to the technical factors if a manual technique was being used for his hip x-ray?  If yes, what adjustment should be made?

- Meet Emma Pear.

  - What classification would her body habitus fall under?

  - Based on her appearance, if a manual technique was being used, what modifications would need to be made for a wrist x-ray?

  - What about for an abdominal x-ray?  If yes, what adjustment should be made?  Are there any other factors to consider?

- A ____________________ pathology will increase the radiopacity of tissue.

- A _____________________ pathology will increase the radiolucency of tissue.

- Why is differential absorption, differential?

- How is beam attenuation defined?  What does it consist of?

- What are scatter interactions?

- What are absorption interactions?

- Are differential absorption and exponential attenuation the same thing?

- Why is scatter radiation such a big deal?

- When the kVp is increased, the total number of interactions in the body are _______________.

- What is a latent image?

- Why is there a difference in intensity of the x-ray beam when the SID is changed?  How is radiographic density affected?

- Calculate the new mAs for each scenario

| Initial mAs | Initial SID | New SID | New mAs |
| --- | --- | --- | --- |
| 50 | 180 cm | 100 cm |  |
| 2.5 | 100 cm | 130 cm |  |
| 250 | 110cm | 150 cm |  |

- The original exposure is 40 mAs at a distance of 72 inches.  The new exposure will be taken at a distance of 60 inches.  To maintain the radiographic density, what will the new mAs be?

- If the intensity of the beam at 100 cm is 80 mR, what would the distance be for the intensity to be 125 mR?

- The original exposure is 75 mAs at a distance of 50 inches.  The new exposure will be taken at a distance of 40 inches.  What will the new mAs be to double the radiographic density?