# APPH 226 – Apparatus and Image Management

# Lab 6 - Grids & Exposure Technique

### Purpose

Objective: 5.3 Evaluate grids and their effect on image quality.

Correlate how grids influence technical factor selection.

### Instructions

Exposure #1

- Enter the patient information into the console, select L-spine.

- Select wall detector.

- Turn AEC off and set 80kVp @ 32mAs.

- Center the tube and detector using an SID of 110cm.

- Collimate to a 20 x 25cm field size.

- Position the body part phantom upright on the cart and as close to the detector as possible.

- Center over the midline of the body part phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Exposure #2

- Maintain the same setup as Exposure #1.

- Move the body part phantom about 15-20 cm away from the detector.

- Center to match the centering used in Exposure #1.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Image Evaluation:

- Describe any differences between the 2 images.

- What type of technique was simulated in Exposure #2?

- Explain which exposure produced the most radiographic density based on the EI/EI_s.

Exposure #3

Grid Ratio: _______

- Select table detector.

- Turn AEC off and set 80kVp @ 32mAs.

- Center the tube and detector using an SID of 110cm.

- Collimate to a 20 x 25cm field size, center over the midline of the body part phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Exposure #4

- Maintain the same setup as Exposure #3.

- Ask an instructor to remove the grid from the table detector.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Image Evaluation:

- Describe any differences between the 2 images.

- Explain which exposure produced the most radiographic density based on the EI/EI_s.

Questions:

- How did the grid affect the amount of exposure that reached the dosimeter?

- An exposure was made without the use of a grid. A repeat exposure is required with a grid. What must be changed to maintain the radiographic density between the two exposures?

- What is the main advantage of using a grid?

- Identify one disadvantage of using a grid.