Worksheet - Window Width and Level – ANSWER KEY

Draw each of the gray scales & answer the following questions.

- 8 bit system    WW:  40    WL: 200

a. How many shades of gray can be displayed in this system?

256 shades of gray

    - Which pixel values will be assigned the color white?

All pixel values ranging from 1-180

    - Which pixel values will be assigned the color black?

All pixel values ranging from 220-256

    - What pixel values will be visualized at these settings?

All Pixel values from 180 – 220 will be assigned a shade of gray for display

- 10 bit system       WW:  600         WL:  700

a. How many shades of gray can be displayed in this system?

1024 shades of gray

    - Which pixel values will be assigned the color white?

All pixel values ranging from 1 - 400

    - Which pixel values will be assigned the color black?

All pixel values ranging from 1000- 1024

    - What pixel values will be visualized at these settings?

Pixel values ranging from 400 – 1000 will be assigned a shade of gray for display

- 12 bit system       WW:  250         WL:  570

  - How many shades of gray can be displayed in this system?

    - shades of gray

  - Which pixel values will be assigned the color white?

All pixel values ranging from 1 – 445

  - Which pixel values will be assigned the color black?

All pixel values ranging from 695- 4096

  - What pixel values will be visualized at these settings?

Pixel values ranging from 445-695 will be assigned a shade of gray for display

- Number of Gray Levels:  4096   Area of interest has pixel values ranging from 50 to 150.

  - What is the dynamic range in bits?

The dynamic range of this system is 212.  12 bit system

  - What is the Window Width?

The WW is 100

  - The Window Level should be set at? The WL set at 100

  - Which pixel values will be assigned the color white? 0-50

  - Which pixel values will be assigned the color black?

150-4096

- Number of Gray Levels: 256

Area of interest has pixel values ranging from 200 – 250

  - What is the dynamic range in bits?

The dynamic range of this system is 28.  8 bit system

  - What should the Window Level be set at?

The WL at 225

  - What is the Window Width?

The WW is 50

  - Which pixel values will be assigned the color white? 0-200

  - Which pixel values will be assigned the color black?

250 - 256

- Number of Gray Levels: 16 384

Area of interest has pixel values ranging from 4500-7800

  - What is the dynamic range in bits?

The dynamic range of this system is 214.  14 bit system

  - What should the Window Level be set at?

The WL at 6150

  - What is the Window Width?

The WW is 3300

  - Which pixel values will be assigned the color grey?

4500-7800

- WW:  50     WL:  500    Dynamic Range:  212

  - If increased contrast is desired on the image what should be manipulated and why?

To increase the contrast the Window Width should be manipulated. Any WW less than 50 will increase the contrast. This will demonstrate fewer shades of grey

  - If decreased brightness is desired on the image, what should be manipulated & what should the new setting be?

To decrease the brightness of the image, the Window Level should be decreased.  Any WL lower than 500 will decrease the brightness of the image

  - If the dynamic range of the system was increased, would this require more computer storage space?

If the dynamic range of the system is increased, larger matrix, more pixels, more data collected more memory or storage needed

  - How many shades of gray can be displayed with this system?

    - shades of gray can be displayed with this system

- WW:  370     WL:  500    Dynamic Range:  210

  - If lower contrast is needed on the image what should be manipulated and why? To decrease the contrast the Window Width should be increased. Any WW greater than 370 will decrease the contrast. More shades of grey will be visualized.

  - If the anatomy to be visualized is lung tissue (air), what should be manipulated and why?

To increase the visibility of the darker densities (higher values on the grey scale), the Window Level should be increased

  - How many shades of gray can be displayed with this system?

1024 shades of gray can be displayed with this system

MRT Program – APPH 226

- Two images are taken on a 12 bit system

Image 1 :  WW – 100, WL - 1000

Image 2:   WW – 250 , WL – 600

      - Which image will display with higher contrast?

Image 1

      - Which image will display with more brightness?

Image 2

      - On Image 1, which pixel values would be assigned the color grey?

950-1050

      - On Image 2, which pixel values would be assigned the color white? 1-475

- Two images are taken on a 10 bit system

Image 1 :  WW – 300, WL - 500

Image 2:   WW – 150 , WL – 200

      - Which image will display with lower contrast?

Image 2

      - Which image will display with more brightness?

Image 2

      - On Image 1, which pixel values would be assigned the color black?

650- 1024

d. On Image 2, which pixel values would be assigned the color black? 275 - 1024

MRT Program – APPH 226