APPH 226 – Apparatus and Image Management

# Lab 13 - Post-Processing

### Purpose

Objective: 7.2 Explain how post-processing affects the image.

Identify post-processing procedures, and understand their functionality.

### Instructions  (Part 1)

Exposure:

- Enter the patient information into the console.

- Select the AP technique for the phantom provided.

- Select Table detector and turn AEC off.  Center the tube and use a 110 cm SID.

- Position the phantom and accurately centre to the body part.

- Place a marker outside of the light field.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor before exposing.

- Take the exposure.

### Instructions (Part 2)

Image:

- Find the rotate/flip function, and manipulate the image into the proper hanging protocol.

- Find the masking function, and try to include the lead marker in the image.

- Find the annotation function, and add any relevant information that would help the radiologist assess the image.

- Find the measuring tool if available, and measure the width of the body part (phantom) on the image.

- Find the magnification function, and magnify the image to better assess the image.

Discussion:

- If a technologist wants to avoid having to use any of the post processing functions, what should they check before taking the exposure?

- Why is the measurement tool important?  Why can’t the technologist just “eyeball” the structures for comparison?