APPH 226 – Apparatus and Image Management

# Lab 11 - Detail and Distortion (continuation of lab 10)

### Phantoms Upper and Lower Extremities

### Purpose

Objective: 5.6 Discuss the effect that geometric factors have on diagnostic quality.

Demonstrate how distortion is influenced by alignment.

### Instructions

Demonstration of Distortion on images seen in Lab 10.

*For exposures 1-5: Use 55kVp @ 1.6mAs with a 110cm SID. Collimate to the edges of the anatomy. Label each exposure with a marker*

Exposure #1

- Position the phantom (upper extremity, lower extremity) lengthwise on the tabletop.

- Center the crosshairs over the middle of the bone.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of the body part on the image: ___________

Exposure #2

- Maintain the position of the phantom.

- Angle the tube 450 cephalad.

- Align the tube and IR, and ensure the bone is centered in the crosshairs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of the body part on the image: ___________

Exposure #3

- Maintain the position of the phantom.

- Angle the tube 300 caudad (in the opposite direction as exposure #2).

- Align the tube and IR, and ensure the bone is centered in the crosshairs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of the body part on the image: ___________

Exposure #4

- Tape the bone on a 450 sponge, so that one end is up higher from the tabletop.

- Align the tube (no angle) and IR, center the bone in the crosshairs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Measure and record the size of the body part on the image: ___________

Exposure #5

- Increase OID by stacking several positioning sponges, but ensure the surface is level. Place the bone on top of the sponges.

- Measure and record the amount of OID used ___________

- Center the bone in the crosshairs.

- Ensure everyone is out of the room & the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure

- Measure and record the size of the body part on the image: ___________

Exposure #6

- Maintain the same setup as Exposure #5.

- Decrease SID to 80cm.

- Adjust collimation to include entire bone.

- Ensure everyone is out of the room & the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure

- Measure and record the size of the body part on the image: ___________

Exposure #7

- Maintain the same setup as Exposure #5.

- Maximize the SID (~142 cm).

- Adjust collimation to include the entire bone.

- Ensure everyone is out of the room & the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure

- Measure and record the size of the body part on the image: ___________

Results:

Object size ____________ mm

| Image | Image size(mm) | Distortion type |
| --- | --- | --- |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |

Image Evaluation Questions:

- Compare images 1-7. How are the different types of distortion visualized? Record observations in the table.

- How can shape distortion caused by improper positioning contribute to size distortion? Is this demonstrated in any of the exposures?

Questions:

- How does SID affect distortion?

- Can angling the beam alone cause size distortion as well as shape distortion?

- Calculate the magnification factor for image 1. 
Image sizeObject size

- Use the equation MF=SIDSOD to calculate the magnification factor for exposure 5. How else can the magnification factor be found? Do both methods give the same result for exposure 5? Explain why or why not.

- Name 3 examinations in which shape and/or size distortion is used intentionally to advantage and how it is accomplished.

Exposure #8

- Add a foot examination.  Set 55kVp @ 2mAs, use an SID of 110cm.

- Place a foot phantom on the tabletop, using a sponge to keep it in an AP position.

- Center over the tarsals, collimate to include one finger width of light around the anatomy.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor, then expose.

Exposure #9

- Maintain the same setup as Exposure #8, but open the collimation as wide as possible.

- Move the foot laterally to one side so the 5th metatarsal at the edge of the light field, still including the entire foot in the collimation.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor, then expose.

Image Evaluation Question:

- What visual differences are seen when comparing exposures 8 and 9?

- What caused the difference to occur? What type of radiographic information can be jeopardized with this error?

Question:

- In order to limit the effects of size and shape distortion on a radiographic image, what must the technologist always ensure prior to an exposure?