APPH 226 – Apparatus and Image Management

Lab 5 continued – Collimation and Scatter

Knee Phantom, Plexiglass and Dosimeter

### Purpose

Objective: 5.4 Manage scatter control and its effect on the image.

Demonstrate the effect of collimation on radiographic density.

### Instructions

Exposure #1

- Enter the patient information into the console and select a knee exam.

- Select free cassette.

- Set 70kVp @ 10mAs.

- Place the dosimeter on the table, in the center of the light field collimated to 20 x 43 cm.

- Move the table detector away to ensure it will not be exposed.

- Place one clear Plexiglass block on either side of the dosimeter.

- Place the phantom on top of the blocks, ensuring there is no weight on the dosimeter. (The dosimeter should be near the center of the phantom)

- Adjust the phantom so that the crosshairs are over the center of the phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter: _________ .

Exposure #2

- Maintain the same setup as Exposure #1.

- Collimate to a field size of 14 x 14 cm.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter: _________ .

Discussion:

- Which exposure should produce more scatter and why?

Exposure #3

- Maintain the same setup as Exposure #2.

- Set 75kVp @ 16mAs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter: _________ .

Exposure #4

- Maintain the same setup as Exposure #2.

- Set 75kVp @ 25mAs.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the results from the dosimeter: _________.

Discussion:

- How does collimation affect radiographic density?

Remember: fewer photons reach the IR, (less density), image will appear lighter, however the system will adjust. Check the exposure indicator.