APPH 226 – Apparatus and Image Management

# Lab 9 - Radiographic Contrast – Tissue Thickness

### Skull Phantom and Plexiglass Blocks

### Purpose

Objective: 5.7 Identify patient variables which may impact the image.

Demonstrate the effect of varying tissue thickness on radiographic contrast.

### Instructions

Exposure #1

- Add a skull exam to the examination.

- Set up for an exposure on the table detector with an SID of 110 cm.

- Select AEC center cell.

- Place a skull phantom on the table and center it in the light field.

- Collimate to include the entire skull.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure and evaluate the image.

- Record the:  EI: _________.

mAs readout: _________.

Exposure #2

- Maintain the same setup as Exposure #1.

- Place 1 piece of Plexiglass under the phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure and evaluate the image.

- Record the:  EI: _________.

mAs readout: _________.

Exposure #3

- Maintain the same setup as Exposure #1.

- Place an additional Plexiglass block under the phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the:  EI: _________.

mAs readout: _________.

Image Evaluation Questions:

- Discuss the tissue thickness, how did it affect the mAs readout? Was there any impact on the radiographic contrast?

- Are there any other differences between the exposures?

- What was the purpose of using AEC for this experiment? What would have occurred if we used a manual technique for all 3 exposures?

- Is there any impact of having decreased radiographic contrast, specifically when assessing these images?

Questions:

- How does tissue thickness affect radiographic contrast? Why?

- Name 3 ways of expressing high contrast?

- Name 3 ways of expressing low contrast.

- What are some methods that can be used to decrease thickness of the part?

- What type of image would provide more information, a high contrast image or a low contrast image?

- What are the benefits of high contrast images?

- What are the benefits of low contrast images?