APPH 226 – Apparatus and Image Management

Worksheet – Density – kVp and mAs

Questions 1-24

- What is the definition of the density of a radiographic image?

- What is the range of OD that can be detected by human vision?

- What are prime factors?

- If there was no option, and a choice had to be made, it is better to take an image that would result in:

- An image with too much density

- An image without enough density

- How is the decision made about whether the radiographic density on an image is sufficient?

- How is visibility achieved?

- What type of relationship does mA and time have with radiographic density?

- Directly proportional

- Inversely proportional

- Inversely squared

- No relationship

- Which mAs setting will produce a radiograph with the most radiographic density?

  - 50 mA, 18ms

  - 500 mA, 3 ms

  - 300 mA, 25 ms

  - 100 mA, 40 ms

- A patient is having difficulty holding still for the exposure.  What would be the best technique to use in this situation?

  - 100 mA, 0.05 s

  - 200 mA, 1/40 s

  - 500 mA, 10 ms

  - 50 mA, 0.1 s

- If all other factors were left unchanged, and the mA is increased from 100 to 200, how will the radiographic density on the image be affected?

- If all other factors were left unchanged, and the exposure time is decreased from 0.1 sec to 0.05 s, how will the radiographic density on the image be affected?

- List five viable combinations of mA and time that will produce 10 mAs.

- Calculate the mAs value that would halve the radiographic density of the original mAs given.

  - 50 mAs =

  - 100 mAs =

  - 240 mAs =

- Calculate the mAs value that would double the radiographic density of the original mAs given.

  - 40 mAs =

  - 75 mAs =

  - 8.4 mAs =

- After an exposure is taken, it is determined that the density needs to be increased slightly, by about 30%.  What should the technologist do?

  - Increase the mAs by 30% for the repeat exposure

  - Increase the mAs by 50% for the repeat exposure

  - Decrease the mAs by 30% for the repeat exposure

  - None of the above.

- What factor of the x-ray beam is controlled by kVp?

  - Quality

  - Quantity

  - Both

  - Neither

- kVp primarily controls _________ .

  - Density

  - Distortion

  - Contrast

  - Detail

- In order to double the radiographic density on an image, what change in kVp is needed?

- The number of electrons in the thermionic emission cloud is controlled by kVp.

  - True

  - False

- X-rays with more penetrability have a ____________ wavelength than weaker x-rays.

  - Shorter

  - Longer

  - Higher

  - Lower

- Calculate what the new kVp should be in order to halve the radiographic density.

Original: 20 mAs and 80 kVp

New: 20 mAs and _____kVp

- The _____% rule is most accurate in the kVp range of __________.

  - 30;  40-80 kVp

  - 30; 60-100 kVp

  - 15; 40–80 kVp

  - 15; 60-100 kVp

- When kVp is increased:

  - The number of x-rays transmitted is increased

  - Compton scattering is increased.

  - The amount of fog is decreased.

  - All of the above

  - A and B only.

- Fill in the missing information on the chart to maintain the same radiographic density.

| Original |  | New |  |
| --- | --- | --- | --- |
| mAs | kVp | mAs | kVp |
| 15 | 60 |  | 69 |
| 40 | 50 | 10 |  |
| 15 | 80 | 30 |  |
| 70 | 90 |  | 77 |