MRAD 256 – Worksheet - Cranium Image Critique – ANSWER KEY

- Positioning:    Tilt or rotation, is the head positioned properly,  are appropriate lines “lined up” properly: OML, IOML, AML etc

- Central ray angulation: what is the angle that should be used for the image/position/projection

- Collimation:  is the collimation appropriate  for the projection?  Comment how it should be corrected

- Required anatomy: is all required anatomy imaged/.  If not, what is missing?  Is the anatomy demonstrated properly ie petrous ridges in lower 1/3 of orbit etc

- Acceptable?: Would you consider this image appropriate for a student logbook?



Positioning: tilt and rotation, both toward the left

Central ray angulation: 15 deg caudad

Collimation: Too large for skull, should only include to base of skull

Required anatomy: yes

Acceptable?: No

Caldwell for skull

Positioning: insufficient chin tuck

Central ray angulation: if OML was perpendicular to the IR, the tube angle was > 15 °

Collimation: ok

Required anatomy: no, petrous ridges are seen below the orbits

Acceptable?: no

Caldwell for skull

Positioning: no chin tuck

Central ray angulation: Should be 15°, if OML was perpendicular to IR, then excessive caudal angulation was used

Collimation: need to collimate tighter side-toside

Required anatomy: no, petrous ridges seen in maxillary sinuses

Caldwell for skull

Case courtesy of Dr Matt A. Morgan, Radiopaedia.org, rID: 35611

Acceptable?: no

Positioning: rotation to the left, places dorsum sellae to the left side of the foramen

Central ray angulation: 30° caudad

Collimation: ok

Required anatomy: yes, dorsum sellae seen in foramen magnum

Townes for skull	Acceptable?: no

Positioning:  not enough chin tuck, OML not perpendicular to IR

Central ray angulation: if OML was perpendicular to the IR, then the tube angle was < 30°

Collimation: ok

Required anatomy: no, dorsum sellae is not seen in foramen magnum, seen superior to it

Acceptable?: no Townes for skull

Positioning:tilted to the right, not enough chin elevation, petrous ridges seen in maxillary sinuses

Central ray angulation: none required

Collimation: includes too much of skull, sinuses require tighter collimation

Required anatomy: maxillary sinuses are not visualized due to superimposition of petrous ridges

Waters for sinuses

Acceptable?: no

Positioning: true lateral as evidenced by superimposed mandibular rami, superimposed orbital roofs. Slight separation of mandibular bodies is due to OID of the side further from the IR

IOML is not parallel to the floor, as occipital bone is very close to the c spine

Central ray angulation: none required, horizontal ray

Collimation:  includes too much inferior to the base of skull, full mandible not required

Required anatomy: yes

Lateral skull

Acceptable?: no, due to collimation size

Case courtesy of Dr Matt A. Morgan, Radiopaedia.org, rID: 37222

Positioning: No tilt or rotation, petrous ridges are seen below the maxillary sinuses

Central ray angulation: horizontal beam, no angle  required

Collimation: Includes full skull, too much included.  Should be collimated to include bottom of maxilla to the supraorbital groove

Required anatomy: all included

Waters for facial bones	Acceptable?:no, due to collimation field size being too large

Case courtesy of Dr Jeremy Jones, Radiopaedia.org, rID: 46272

Positioning: tilted to the right and rotated to the left, not enough chin tuck

Central ray angulation: should be 15◦, if OML was perpendicular to the IR then the angle is >30 degrees

Collimation: includes too much below the maxilla, due to incorrect positioning/ angle Required anatomy: no

Caldwell for facial bones

Acceptable?: no

Case courtesy of Dr Bruno Di Muzio, Radiopaedia.org, rID: 48270

Positioning: IOML is parallel to floor, slight tilt evidenced by one EAM and TMJ slightly superior to the other

Central ray angulation: horizontal beam

Collimation: includes too much posterior to the EAM

Required anatomy:yes

Lateral for facial bones

Case courtesy of Dr Bruno Di Muzio, Radiopaedia.org, 	Acceptable?:no

rID: 48270

Positioning: rotated to left, as evidenced by shorter distance from left orbital rim to skull table compared to the right, petrous ridges seen below maxillary sinuses indicating proper elevation of MML

Central ray angulation: horizontal beam

Collimation: should be collimated only to include bottom of maxilla to supraorbital groove

Waters for facial bones

Required anatomy: yes

Case courtesy of Dr Bruno Di Muzio, Radiopaedia.org, rID: 48270

Acceptable?: no

Positioning: true lateral as evidenced by superimposed orbital roofs, clearly defined sella turcica, superimposed mandibular rami

Central ray angulation: horizontal

beam

Collimation: does not need to include all of mandible, assume the side to side collimation is correct (black screen added for labelling)

Required anatomy: all sinuses are included

Lateral sinuses

Acceptable?:no

Positioning: OML perpendicular to IR

Central ray angulation: horizontal

beam

Collimation: good top to bottom, and assume black screen side to side was for labelling

Required anatomy: all included

PA mandible

Acceptable?: YES!

Positioning:  Positioned to include all of down side mandible free of superimposition, chin extended to clear ramus from c spine

Central ray angulation: 0 deg tube with 25 deg head tilt, OR 0 deg head tilt with 25 deg cephalad tube angle OR 15 deg tube angle and 10 deg head tilt (or vice versa)

Collimation: good

Axiolateral oblique mandible	Required anatomy: yes, condyle to

mentum is demonstrated free of

superimposition of other half of mandible

Acceptable?: yes