RADIOGRAPHIC TECHNIQUE 1

# Lab “Mock” & Midterm: Testable Projections

# Hand – PA, Oblique, Fan lateral Thumb - AP, Oblique, Lateral Wrist – PA, Oblique, Lateral

Scaphoid – PA axial 15 degrees with ulnar deviation

Forearm – AP, Lateral

# Elbow – AP, AP internal oblique, AP external oblique, lateral

Humerus – AP with external rotation, lateral

# Shoulder – AP with external rotation (to include entire scapula and clavicle) AP glenoid fossa, trans scapular Y, Superoinferior axial

Clavicle – AP, 15 °AP Axial

Requisition notes:

- Only perform the projections specified on the requisition

  - Examples:

    - “Scaphoid – Axial only”; only perform the 15o PA axial with ulnar deviation projection

    - “Hand – Oblique & Lateral only”; do not perform the PA hand projection

    - “Elbow – Oblique’s only”; only perform the AP internal oblique and AP

external oblique projections

- If the requisition specifies “Routine” you must perform all routine projections for the

specified body part

  - Example:

    - “Shoulder – Routine”; you must perform all four routine projections as per lecture (and your PowerPoint): AP with external rotation, AP Oblique (aka. Glenoid), PA Oblique (aka. Scap Y), & Superoinferior axial