Purpose

APPARATUS & IMAGE MANAGEMENT

# Lab 8 - Repeat & Reject Analysis – ANSWER KEY

Objective: 10.4 Perform a repeat/reject analysis. Analyze the results of a repeat/reject analysis.

Instructions

- With your group, select images.

- Categorize each image by deciding the reason for the repeat/reject.

- Record the data on the chart provided.

- Calculate the causal repeat rate for each category, and record the results on the chart.

Questions:

- What is the total repeat rate, if the total number of views performed was 387?

(# of repeats) / (total # of views) X 100

- Is the repeat rate acceptable?

The rate should be somewhere between 3 and 10 %.

- Calculate the causal rate for improperly exposed films

(# of repeats for specific reason)/(total # of repeats) X 100% (underexposed and overexposed added together)

- What are 5 factors that can affect the total repeat rate?

  - Equipment

  - Radiologist

  - Technologist skill level

  - Type of patients

  - Processing equipment

  - Shift

- What is the difference between a causal repeat rate, and a total repeat rate?

Causal rate is specific to the reason for the repeat. The total repeat rate is the percentage of all the views taken, that needed to be repeated.

- What does a total repeat rate of 2% imply?

Very highly trained dept. Little cooperation from technologists, radiologist is very accommodating. Issues with the data collected.

- What is the difference between a repeat and a reject image?

Rejects are scrap, and they do not result in a repeat exposure to the patient. A repeat is a film that resulted in the patient being exposed again.

1