# Worksheet: X-ray production Review - Answers

## Purpose:

Objective 1.1: Explain the process of x-ray production in an X-ray tube.

## Instructions:

Identify the assemblies, and path of electron travel; Label the parts of the X-ray tube.

*Port = Window

Instructions:

Answer the multiple-choice questions.

- The negative side of the x-ray tube holds the:

  - Filament

  - Target

  - Rotor

  - Cathode

- What causes thermionic emission?

  - Rotation of the target

  - Current variation

  - Potential difference

  - Heating the filament (enables electrons to “boil” off the filament)

- What is a characteristic of a Target used in conventional CT scanners?

  - Lightweight

  - Fast cooling rate

  - Prolonged tube life

  - Fixed (conventional = old; in older tubes, the target did not rotate)

(Thicker targets enable ↑ heat capacity so we can take more exposures for a longer period, in CT)

(Thicker targets enable ↑ heat capacity so we can take more exposures for a longer period, in CT)

Why does CT use a thicker Target, than GR x-ray tubes?

  - Enables higher target rotation speeds

  - Decreases spatial resolution and heat dissipation

  - Increases the heat load on the anode target

  - Exposures are longer and produce more heat

- What is an important consideration in CT imaging?

(Longer exposure times ↑ heat production; should always consider how much time you have available to scan, to ensure you can image all required anatomy)

(Longer exposure times ↑ heat production; should always consider how much time you have available to scan, to ensure you can image all required anatomy)

Housing cooling times

  - Anode cooling times

  - Maximum exposure times

  - Maximum patient dose

- Where are the useful photons emitted?

  - Filament

  - Target

  - Focusing cup

  - Focal spot (the area the electrons “strike” the target)

- What enables maximum emission of x-ray photons from the tube, with the least amount of absorption?

  - Filament

  - Window (aka. port – a thinner section of tube envelope, located below the target)

  - Focusing cup

  - Target

- When electrons bombard the target, how much of their kinetic energy converts to heat?

  - 1%

  - 25%

  - 50%

  - 99%

- What is a function of an x-ray tubes protective housing?

  - Prevents electric shock

  - Dissipates heat

  - Maintains tube current

  - Prevents arcing