MRAD 302- Specialized Imaging 2

Learning Material and Worksheet - Angiography and Interventional Radiography

Learning Outcomes

Explain the characteristics of interventional imaging equipment

Explain the basic concepts of interventional imaging

Determine the best patient care plan for an interventional patient

Explain an interventional procedure

Historical Development

In January 1896, just 10 weeks after the announcement of Roentgen's discovery of x-rays, Haschek and Lindenthal announced that they had produced a radiograph showing the blood vessels of an amputated hand using Teichman's mixture, a thick emulsion of chalk, as the contrast agent. This work heralded the beginning of angiography. The advancement of angiography was hindered, however, by the lack of suitable contrast media and low-risk techniques to deliver the media to the desired location. By the 1920s, researchers were using sodium iodide as a contrast medium to produce lower limb studies comparable in quality to studies seen in modern angiography.

The first human cardiac catheterization was reported in 1929 by Forssman, a 25-year-old surgical resident who placed a catheter into his own heart and then walked to the radiology department where a chest radiograph was produced to document his medical achievement. Catheterization of the heart soon became a valuable tool used primarily for diagnostic purposes. Through the 1940s, the basic catheterization study remained relatively uncomplicated and easy for physicians to perform; however, the risk to the patient was significant.

Until the 1950s, contrast media were most commonly injected through a needle that punctured the vessel or through a ureteral catheter that passed into the body through a surgically exposed peripheral vessel. In 1952, shortly after the development of a flexible thin-walled catheter, Seldinger announced a percutaneous* method of catheter introduction. The Seldinger technique eliminated the surgical risk, which exposed the vessel and tissues Selective coronary angiography was first reported by Sones in 1959, when he inadvertently injected contrast media into the right coronary artery of a patient who was undergoing routine aortography. In 1962, Ricketts and Abrams described the first percutaneous method for selective coronary angiography. This method was further perfected in the late 1960s with the introduction of preformed catheters designed to engage the ostium of the right and left coronary arteries.

Pioneers in the field overcame equipment obstacles. It has been said the “Fathers of

Interventional Radiology and Cardiology” were Charles Dotter and Andreas Gruntzig, respectively. Dotter performed the first successful dilation of a superficial femoral artery in 1964 using coaxial catheters. Percutaneous transluminal angioplasty (PTA) was described in Circulation in November, 1964 with co-author Dr. Melvin Judkins. In 1966, Dotter fabricated a reinforced balloon dilating catheter, but it was not used on patients. Dr. Werner Portsman (Berlin, Germany) introduced “Korsett Balloon Kather” an 8 French outer Teflon catheter with four longitudinal slits in 1973. A latex balloon catheter was inflated inside the longitudinal slits. In September 1977, Dr. Andreas Gruntzig used successfully used a balloon PTA to treat a left anterior descending coronary artery stenosis. Gruntzig and Hopff introduced the double-lumen, balloon-tipped catheter. One lumen allows the passage of a guidewire and fluids through the catheter. The other lumen communicates with a balloon at the distal end of the catheter. When inflated, the balloon expands to a size much larger than the catheter. Double-lumen, angioplasty balloon catheters are available in sizes ranging from 3 to 9 French, with attached balloons varying in length and expanding to diameters of 2 to 20 mm or more Transluminal angioplasty can be performed in virtually any vessel that can be reached percutaneously with a catheter). In 1978, Molnar and Stockum described the use of balloon angioplasty for dilation of strictures within the biliary system. Balloon angioplasty is also conducted in venous structures, ureters, and the gastrointestinal tract.

The 1980s saw the development of expandable metallic stents. Andrew Cragg, Charles Dotter, Cesare Gianturco, Dierk Mass, Julio Palmaz, and Hans Wallsten were the first stent pioneers. These stents were composed primarily of a stainless steel alloy or thermal memory stents made of nitinol (alloy of nickel and titanium) and were self or balloon expandable. Three stents available for use in 1985 were the Gianturco Z, Palmaz, and Wallstent. Dotter continued advancement of cardiovascular procedures by first using Streptokinase for selective thrombolysis. In the 1980s, Urokinase was used for this widely performed procedure. Continued growth of thrombolysis was due to the advancement of fibrinolytic agents (recombinant tissue plasminogen activators).

Therapeutic vascular occlusion procedures began in 1931 with an open surgical embolization of a carotid cavernous fistula. Dr. Shoji Ishimore used Gelfoam pieces through a polyethylene tube into an exposed carotid artery. Embolization became popular during the 1980s and 1990s with advancement of embolization agents, such as gelatin sponges (Gelfoam), polyvinyl alcohol (Ivalon), liquid and rapidly solidifying polymers including cyanoacrylate glue, coils, and detachable balloons.

Early angiograms consisted of single radiographs or the visualization of vessels by fluoroscopy. Because the advantage of serial imaging was recognized, cassette changers, roll film changers, cut film changers, and cine and serial spot-filming/digital devices were developed. Until the early 1990s, most angiograms recorded flowing contrast media in a series of images that required rapid film changers or cinefluorography devices; however, presently digital subtraction angiography (DSA) systems are used almost exclusively. Although some institutions may still have rapid film changers, most often the filming technique is by DSA. The newer imaging equipment has much better image quality and can produce images at a rate of 30 frames per second. In addition, digital imaging is cost-effective because images are stored electronically, reducing the need for expensive film and film storage. Digital images can be archived and retrieved in seconds from within the institution or any network connection. DSA imaging provides the interventionalist with a variety of tools for image manipulation analysis and measurement.

The resolution possible with early digital equipment was a drawback to the use of digital imaging in the cardiovascular laboratory. Larger matrix size, the obvious solution to this problem, allowed for acceptable resolution but also created another problem: how to acquire and store large volumes of digital information. In the late 1970s and early 1980s, the high-speed parallel transfer disk was introduced to solve the acquisition and short-term storage problem. This new disk acquired and stored an entire coronary angiogram and made real-time digital playback during the procedure possible. Permanent storage of the digital images remained a problem, however. Floppy disk and computer tape storage were inadequate solutions because they required significant time and supplies. Long-term storage of large amounts of digital images has benefited from advances in computer technology, which provide high-speed, large-capacity methods of storage, capable of acquiring large amounts of data (terabytes) with very high resolution. A problem incumbent with digital imaging was the incompatibility of the storage media from one system to the other. Today, networking, security, redundancy, and image integrity are issues for laboratories equipped with digital technology.

The 1960s and 1970s brought tremendous advances in radiologic and cardiovascular medicine and technology. Radiographic imaging and recording equipment, physiologic monitoring equipment, and cardiovascular pharmaceuticals and supplies became increasingly reliable. The uses of computers in the cardiovascular interventional laboratories have facilitated the development of this rapidly growing subspecialty of the cardiovascular medical and surgical sciences. These advances and trends have enabled angiography to evolve from a simple diagnostic investigation to its current state as a sophisticated diagnostic study and interventional procedure.

Interventional Radiography (IR) refers to radiological procedures that intervene in a disease process, providing therapeutic outcome. Procedures are categorized as vascular or nonvascular.

Angiography refers to a radiologic imaging of blood vessels after an injection of contrast medium for treatment of disease and also to provide diagnostic information.

Generally performed in an angiographic suite under direction of interventional radiologist using fluoroscopy. There has been increase in complexity of procedures and interventional exams are now performed in the operating room to minimize risk of infection and allow for rapid surgical management in case of complications.

Pre-Procedural Care of Patient

- Solid food withheld for approximately 8 hours prior to procedure. Fluid intake is encouraged, along with daily medications (exception is blood thinners).

- Signed informed consent form—ensures patient understands risks and possible complications of procedure.  o Completed by interventional radiologist prior to exam, usually when consult is performed.

- Explain procedure thoroughly to patient (length of time, contrast sensation, breathing instructions, etc.).

  - Explain risks and complications and what is required from the patient during the procedure and what the patient can expect to experience. o Important that patient understands what is happening, as success of test depends on patient cooperation.

  - Constant communication and monitoring of patient throughout exam from all members of the IR team is key.

- Patient’s medical history obtained (allergies, cardiac history, renal function, etc.).

- Essential that interventional patients complete a pre-procedure blood workup.

  - Creatinine levels can measure the risk associated with nephrotoxicity related to iodinated contrast media.

- Some procedures in interventional radiography carry a risk of internal bleeding, therefore, international normalized ration (INR) and platelet levels need to be kept at a level that minimizes the chance of a bleed occurring.

- Patient should be well hydrated:

  - Clear liquids up to 2 hours prior to procedure for outpatients and possible IV hydration for inpatient.

  - Some patients may need to fast 6–8 hours prior to procedure. o Prior to test, the physician may alter medications.

  - Lab work complete within one month of exam.

  - Patient must arrange for transportation post-exam if sedation is used.

- Assess peripheral pulses in feet (dorsalis pedis pulse/top of foot and posterior tibial pulse/inner ankle).

- An IV will be started, baseline vital signs obtained and monitored throughout procedure, warm blankets given (patient comfort=cooperation).

- Procedure site is shaved, cleaned and sterile drape applied.

- Mild sedative may be administered pre-exam to help decrease patient anxiety and help them to relax (e.g., Ativan, diazepam, phenobarbital, etc.).

- Analgesics may be given to reduce pain (e.g., meperidine, fentanyl, morphine, etc.).

General Precautions with Contrast Administration

Before Injection

- Discuss the procedure with the patient

- Obtain informed consent

- Establish baseline set of vital signs

Know the Patient

- Check the chart for history of allergies or hypersensitivity.

- Check the results of patient’s lab work:

  - Blood urea nitrogen (BUN) o Creatinine

  - Prothrombin time (in cases of arterial access) o INR

  - Platelet count o Partial thromboplastin time

- Check whether the patient has other medical problems:

  - Hepatic or renal disease o Pregnancy o Multiple myeloma o Congestive Heart Failure o Graves’ Disease

  - Homozygosity for sickle cell disease o Known or suspected pheochromocytoma (solid tumors of neurologic origin) o Severe cardiovascular disease, bronchial asthma or hyperthyroidism

Know the Procedure

- Contraindications and limitations to the specific special procedures.

- Know the possible reactions that can occur when contrast agent used.

- Check emergency equipment.

- Know location and contents of crash cart.

Post-Injection

- Know how the physician can be reached.

- Evaluate patient’s vital signs for abnormalities.

o Respiration, pulse, blood pressure, presence of cyanosis, etc.

- Remain with patient for at least 20 minutes after injection.

Reactions to Contrast Media

- Happens within 5–15 minutes of injection.

- Ensure patient is properly hydrated to avoid renal failure.

- Overdose reactions—infants, adults in acute renal failure, adults with cardiac failure and adults with hepatic failure are at risk.

- Anaphylactic reactions—allergic reactions caused by hypersensitivity to the contrast media.

  - May be severe or mild and are easily recognized and treated. o May have had previous reaction, ensure you document it.

  - May be treated with Benadryl intravenously or intramuscularly.

Post-Procedural Care

- If a procedure such as a tube change is performed, the patient will be given literature to take with them but nothing requires them to remain in the department.

- Post-care for vascular procedure is more involved (i.e., post-angiogram).

- Pressure is applied to patient’s puncture site for at least 20 minutes and reassessed.

- The extremity is checked for warmth, colour and numbness to ensure circulation has not been disrupted.

- Pressure is often supplied by a vascular closure device as the patient usually receives heparin during the procedure.

- Patient is kept on bed rest for a minimum of 4 hours post-procedure.

- Vital signs are monitored about every 15 minutes.

- Patient is to be kept quiet and still (to avoid re-opening puncture site).

- Patient is hydrated through IV (large amounts of contrast given during procedure, so need to help kidneys flush out contrast).

- Information about procedure is recorded, such as contrast amount given, medication administered, vital signs, radiologist, type of exam, images taken, etc.

Arterial Puncture Techniques

Interventional radiography is accomplished by accessing a vessel, introducing a catheter into its lumen and manipulating it so that the catheter arrives at the desired location. The arterial approach is used most often, but veins can be accessed as well.

Most interventional radiographic procedures utilize the percutaneous puncture of the specific vessel using the Seldinger technique or Modified Seldinger method:

- Seldinger punctures a hole in the anterior and posterior wall of the vessel.

- Modified Seldinger punctures a hole in the anterior wall only.

Establishing Vessel Access

- The arterial pulse is identified and the puncture site is chosen.

- The site is shaved, surgically prepared and draped.

- Local anesthetic is applied to the area and a cut is made in the skin to facilitate needle access.

- An 18g Seldinger needle is inserted into the vessel, the stylet is removed and a pulsatile blood flow is sought.

- A guidewire is inserted through the lumen of the needle and directed to the desired position.

- Once in position the needle is withdrawn.

- The catheter is then threaded over the guidewire and advanced to the area of interest under fluoroscopic control. A hand injection of contrast ensures correct placement.

- Removal of guidewire from inside the catheter.

- Catheter remains in place as connection between exterior of body and area of interest.

- During the procedure, the catheter is flushed with heparinized saline (if not contraindicated) to prevent thrombosis.

- At the conclusion of the study, the catheter is removed and pressure is maintained at the site until hemostasis is achieved (at least 20–40 minutes).

Several approaches are used for arterial catheterization:

- Femoral artery  o Most preferred, easy access, large vessel

o Contraindicated if previous surgical grafts, presence of aneurysm or occlusive vascular disease

- Brachial

- Axillary artery

- Radial

Femoral approach

- Can be done antegrade or retrograde. Antegrade is less desirable in obese patients as there is increased difficulty maneuvering the needle around large abdominal folds.

- The femoral artery is palpated carefully because the inguinal ligament is close to artery and if ligament is punctured the patient will react.

- The needle is inserted between the index and middle fingers at an angle of 25–30 degrees toward the midline and should correspond to the longitudinal axis of the vessel.

- The right or left femoral artery can be used (the right is the most common).

- Ultrasound is used to locate the access point of the needle to ensure no lesion or calcification present.

Catheterization is preferred to needle injection of the contrast for the following reasons:

- The risk of extravasation is reduced.

- Most body parts can be reached for selective injection.

- The patient can be positioned as needed.

- The catheter can be safely left in the body while images are being reviewed by the interventional radiologist.

Potential complications of an interventional procedure include:

- Vasovagal reaction  o Triggered when contrast stimulates the vagus nerve, causing cardiovascular changes that result in increased vasodilation of arterioles.

o Signs and symptoms include diaphoresis, hypotension and sinus bradycardia.

- Bleeding at the puncture site (controlled by applying pressure to site).

- Thrombus formation could disrupt blood flow to distal parts.

- Mild or severe allergic reaction to the contrast.

- Embolus formation (piece of plaque dislodged from vessel wall may lead to CVA or vessel blockage).

- Dissection of vessel (catheter tears intima of vessel).

- Infection at puncture site (contamination of sterile field).

Vessel Puncture Closure Devices

Arterial puncture sites can be sealed by manual compression or mechanical means.

- Manual compression should be done for a minimum of 20 minutes.

- It’s important to maintain the peripheral pulse throughout compression.

- If after 20 minutes bleeding is still apparent, compression should be continued for another 20 minutes.

- Mechanical compression includes compression cuffs, weights or specially designed clamps. o Effective for patients taking anticoagulants.

Radiation Protection

- Lead aprons, thyroid shields and lead glasses.

- Lead gloves (sterile surgical gloves lined with lead).

- Limit fluoroscopy time (keep as low as possible and pulsed).

- Precise collimation.

- Ceiling-mounted clear lead shields to protect radiologist during fluoroscopy.

- Patient protected by filtration totaling no less than 2.5 mm of aluminum.

- Avoidance of repeat exposure through clear instructions to patient.

- Minimize SID by positioning detector as close to patient as possible.

Interventional Procedure Catheters and Guide Wires

Catheters are designed differently depending on their purpose. The 3 main characteristics of catheters include:

- Shape

- Maneuverability/torque

- Maximum injection rate

Catheters are made of a pliable plastic, which allows them to straighten for insertion over a guide wire. Catheters come in a variety of styles/sizes:

- Can be predetermined design or shape to be maneuvered into the origins of vessels for selected injections.

- Catheters can have 1 or multiple holes, depending on the volume and rate of contrast being injected.

- Common catheter size ranges from 4 Fr to 7 Fr. Smaller and larger catheters available if required.

- Catheter measurement (French or Fr) is the outside diameter.

Guide wires are made of a solid wire core (mandrel) covered by stainless steel. They are used in interventional procedures as a “platform” over which the catheter is to be advanced.

- The position of the guide wire is fixed and the catheter advanced over it until it meets the tip of the guide wire.

- Guide wires come in various sizes, shapes and lengths and care must be taken to match the proper guide wire to the selected access needle and catheter.

- The stiffness and flexibility of the guide wire can vary based on the design and materials used.

- More flexible guide wires are able to transverse through tortuous anatomy.

Note: a vascular sheath is used to protect the access point and allow the exchange of various catheters if necessary during a procedure. The Fr measurement of the sheath is the inside diameter.

Interventional Procedures

Percutaneous Transluminal Angioplasty

Percutaneous transluminal angioplasty (PTA) is a nonsurgical procedure involving the access of a blood vessel through a puncture in the skin and the performance of a procedure through the blood vessel in order to reshape or repair the inside diameter of the vessel through balloon inflation. The objectives of this procedure are to:

- Improve the blood flow (increase in lumen size).

- Maintain long-term vessel patency.

- Create a smooth inner surface of the vessel.

All of these objectives are completed with minimal disturbance of the vascular wall structure. The procedure should permanently improve blood flow in a vessel and result in minimal complications.

Percutaneous Transluminal Angioplasty Procedure

- Technique of catheter introduction during PTA is the same used for routine angiography.

- Usually a routine angiogram is performed to define the anatomy.

- The angioplasty guide wires are advanced through the lesion and then the appropriate catheter is advanced over the wire and through the lesion. (This step is critical and can be challenging due to anatomic variations. Most complications occur at this stage.)

- The balloon is then inflated and dilation of the vessel occurs.

- The amount of force required to open the vessel successfully varies with the type of lesion and its flow characteristics.

- The amount of force applied should be sufficient to achieve the objectives of the angioplasty. The inflation of the balloon is done under fluoroscopic guidance.

- Each balloon has a set amount of pressure that needs to be applied during the inflation of the balloon and a set amount of time that the balloon should be inflated for. (This information is provided by the manufacturers.)

- Balloon catheters have been designed to provide maximum dilation force on inflation and enhanced ability to cross the blockage.

- Balloon catheters have multiple levels of inflation so that catheter exchange during the procedure is minimized (so more than 1 part of the vessel can be dilated with the balloon catheter, depending on the amount of vessel occlusion).

- After dilation, a post-procedural angiogram is usually performed to assess the results. If dilation is successful, then the catheter is removed.

Risk of Thrombosis and Treatments

The theory is that foreign materials (i.e., catheters and guide wires) introduced into the body are thrombogenic in nature (can cause a thrombus/blood clot). A PTA is disturbing to the vascular system and its flow pattern creates a situation in which complicating thrombosis is a possibility.

Pre-procedural administration of antithrombotic agents usually reduces the likelihood of thrombus formation (e.g., Aspirin, heparin, warfarin). The method and the length of time these meds are administered is dependent on the patient and physician. Some pharmaceuticals are administered during the procedure and some are administered long term (1–6 months).

Interventional Techniques Used to Reduce Blood Flow

Procedures for reducing blood flow include:

- Embolization

- Balloon occlusion

- Intravascular infusion of vasoconstrictors

- Intravascular electro-coagulation

Most of these techniques are performed using percutaneous puncture of the femoral or another

artery but occasionally, the embolic material is inserted directly into the vessel via an arteriotomy (any surgical incision into the lumen of an artery).

Indications and Contraindications for Embolization:

- Post-traumatic hemorrhage.

- For occlusion of the blood supply to highly vascular neoplasm.

- For reduction of bleeding during and after surgical procedures.

- Because transcatheter embolization is presently used in high-risk cases, there are no definitive contraindications to the procedure.

- Ideally, the embolization method chosen reduces the flow of blood without incurring ischemia of the surrounding tissue.

Transcatheter Embolization and Balloon Occlusion

The search for the ideal embolic material has yielded many products, but none exhibits ALL the characteristics of an ideal substance. An ideal embolic material would be:

- Nontoxic

- Stable and insoluble in the vascular system

- Radiopaque

- Capable of being shaped and sterilized

- Capable of introduction via an angiographic catheter

Currently, the material used is determined by:

- Pathology

- The patient’s clotting ability

- The nature of the blood supply

Occasionally, it is necessary to use a combination of more than one type of embolic material to successfully isolate an area. The substance used to occlude blood vessels can be categorized by how long they remain in a vessel—temporary or long term.

Temporary occluders include Gelfoam, autologous blood clots (i.e., fat), muscle tissue clots and balloon catheters. These temporary occluders are either mechanically removable, such as balloon catheters, or they are quickly absorbed by the body through intravascular lysis.

Long term agents include coils, detachable balloon catheters and polyvinyl alcohol foam.

Balloon catheter occlusion is a safe, temporary method of occluding vessels to control or prevent bleeding. It is also used to isolate vessels for selective infusion of chemotherapeutic agents or placing other types of embolic media. Advantages of balloon catheter occlusion:

- The catheters are retrievable for removal.

- The position of the balloon can be changed easily.

There are 4 types of balloon available. The details are not going to be discussed here, but note that there are single and double lumen balloons.

Occluding Spring Emboli and other Mechanical Occluders

Occluding spring emboli are devices of stainless-steel coiled wire about 5 cm long and available in varied diameters. These are considered permanent because they remain in the vessel for more than 30 days.

Other mechanical occluders are:

- The bristle brush. It resembles a miniature chimney cleaning brush.

- Liquid embolic media—this includes “tissue adhesives” (polymerizing agents). They are long term and these materials are used in life-threatening cases. They solidify when they come into contact with tissue, thus forming a permanent embolus that seals the vessel almost immediately.

Complications of transcatheter vascular embolization include:

- Localized transient pain

- Mild fever and malaise

- Increased pulse and respiration rates

- Restlessness and irritability

- Loss of appetite and insomnia for a period of 24–48 hrs

Other complications may occur such as:

- Embolization of non-target organs

- Ischemia

- Nerve palsy

- Infection

- Possible death

It should be remembered that these procedures are performed on patients who present a great surgical risk and on patients who are in life-threatening situations. As with any radiographic procedure, it should be determined that the benefit of performing an interventional procedure outweighs the risks presented by the procedure.

Interventional Techniques Used to Increase Blood Flow

Procedures for increasing blood flow include:

- Percutaneous transluminal angioplasty (PTA)

- Vascular stent placement

- Intravascular thrombolysis

- Intra-arterial infusion of vasodilators

These procedures provide relatively safe methods of increasing blood flow without the necessity of extensive surgery.

Indications and Contraindications

- Dilate vessel stenosis

- Re-canalize obstructed vessels

There are no defined contraindications to the use of increased blood flow techniques. In most cases, patients are usually candidates for surgery and attempts at treatment with these techniques usually outweigh the risks involved.

Questions

- Give an example of a selective angiogram.

- Where is the puncture location for a high approach translumbar angiogram?

- Describe the steps involving a Seldinger puncture. What’s the difference between a Modified Seldinger puncture and a Seldinger puncture?

- A vertebral artery angiogram demonstrates the blood flow to what part of the brain?

- What three branches originate off the aortic arch?

- Name the 2 most common projections performed to demonstrate an internal carotid artery.

- What 2 arteries unite to form the basilar artery?

- Who performs a coronary angiogram?

- What three arteries originate off the celiac trunk?

- What does a peripheral angiogram demonstrate?

- Where is the location of the popliteal artery approximately?

- Which type of angiogram demonstrates the brachial artery?

- An angiogram is a diagnostic procedure and an interventional procedure is a ____________ procedure.

- What type of a procedure is an angioplasty?

- How does a balloon occlusion procedure influence blood flow?

- What is an  indication for embolization?

- What can be used to reduce blood flow?

- What is the purpose of an angioplasty procedure?

- What interventional procedure that prevents emboli from traveling to the pulmonary vessels?

References

Bontrager, K. L., & Lampignano, J. P.current yearTextbook of radiographic positioning and related anatomy current edition St. Louis, MO: Mosby Elsevier.

Ehrlich, R. A., & Coakes, D. M. current year Patient care in radiography: With an introduction to medical imaging (current edition.). St. Louis, MO: Mosby Elsevier.

Long, B. W., Hall Rollins, J., & Smith, B. J. (current year). Merrill's Atlas of radiographic positioning and procedures (current edition.). St. Louis, MO: Mosby Elsevier.

Snopek, A. M. current year). Fundamentals of special radiographic procedures (current edition.). St. Louis, MO: Saunders Elsevier.