# Mandible:

## PA:

- 80-90 kV

- Distances from the mandibular rami to lateral aspects of cervical vertebrae are equidistant

- Width of mandibular rami are symmetrical (no rotation)

- Mandibular rami and lateral portion of the body seen free of superimposition

- Anterior body and mentum are superimposed over the cervical spine

- TMJ’s are visualized in the same horizontal plane, seen through mastoid processes (no tilt)

## 35° Axial Townes:

  - 80-90 kV

  - Collimated to include mandibular rami and condyles (condyles to gonions)

  - TMJ’s included and are demonstrated in the same horizontal plane

  - Minimal superimposition of petrous ridge on condyles

## Axiolateral Oblique:

- 75-85 kV

- No overlap of the ramus on the cervical spine

- Ramus, body and mentum of side of interest included in collimation

- No superimposition of the up side over required anatomy

# TMJ’s

## 35° Axial Townes:

  - 80-90 kV

  - Collimated to TMJ’s only

  - TMJ’s are demonstrated in the same horizontal plane

  - Minimal superimposition of petrous ridge on condyles

## Axiolateral TMJ (Schuller method):

  - 80-90 kV

  - Demonstrates TMJ in open and closed mouth positions

  - Closed: posterior teeth in contact, condyle visualized in the temporomandibular fossa

  - Open: mouth open wide, condyle visualized anterior and inferior to the temporomandibular fossa

  - TMJ of interest demonstrated free of superimposition of upside anatomy

## Panorex:

- Requires orthopantomography unit

- 85-95 kV

- Condyles perpendicular to the bottom edge of the IR and equidistant from the coronoid process

- Right and left structures will be symmetrical