MRAD 302 – Specialized Imaging 2

Magnetic Resonance Imaging (MRI) Worksheet – ANSWER KEY

Purpose:

This activity will help you gain knowledge of alternative imaging procedures, specifically MRI.

Instructions:

Review the following information and fill in the answers where indicated.

MRI Imaging Basics

- The human body consists of approximately 85 % water.

- Water consists of hydrogen and oxygen atoms. MRI primarily images hydrogen atoms which are found in great abundance in human tissue.

- The hydrogen atom consists of positively charged proton and negatively electron. The proton is electrically charged and spins around its axis. The hydrogen proton can be looked at as a tiny bar magnet with a north and south pole. These tiny magnets are randomly positioned in our bodies and the magnetic force is equalized.

Steps in acquiring an MRI image:

- The patient is placed in a strong uniform magnetic field. Hydrogen atoms align and have a net magnetization along the axis of the magnetic field. They precess (spin around their own axis) at the same rate (speed) but they are out of phase. The speed at which they precess is directly proportional to the strength of the external magnetic field.

- Having the protons align does not create an image. Control interface, radiofrequency transmitter, receiver, analog to digital converter, computing and display are the fundamental steps in MRI imaging.

Excitation:

1. When a radiofrequency (RF) is applied at the precession (resonance) frequency of the aligned protons, the protons can absorb that energy. The protons are now lifted into a higher energy state. This rotates the net magnetization. The protons also begin to precess “in phase.”

Relaxation:

- Protons do not like being in this higher energy state, so when the RF is turned off the protons want to relax to their original state.

- When the RF is turned off:

  - The protons release the absorbed energy in the form of RF waves. Each tissue will relax (release energy) at different rates. The protons return to the Z axis. This is called. T1 relaxation

  - The protons begin to spin out of phase again. The process of getting from total in phase to a total out of phase situation is called T2 relaxation

Remember:

- T1 and T2 relaxation are two independent processes happening simultaneously.

- T1 happens along the Z axis, T2 happens along the X-Y axis.

- T2 is quicker than T1.

|  | T1 (ms) 0.25T | T1 (ms) 1.5T | T2 (ms) |
| --- | --- | --- | --- |
| Grey matter | 530 | 921 | 101 |
| White matter | 422 | 787 | 92 |
| Kidney tissue | 417 | 652 | 58 |
| Kidney tumor | 733 | 907 | 83 |
| Liver tissue | 250 | 493 | 43 |
| Liver tumor | 713 | 905 | 84 |

Acquisition:

During the relaxation phase the protons shed excess energy in shape of radiofrequency waves. These waves are picked up with a receiver coil/antenna.

Computing:

The received RF waves emitted by the protons are then fed to a computer that converts the information into a digital image.

Safety Considerations for MRI

Due to the presence of a strong magnetic field, application of external RF and enclosed spaces, safety considerations in MRI include:

- Projectile effect: A static magnetic field surrounds the scanner at all times (fringe field). If metal objects interact they can become projectiles. Small objects can reach high speeds up to 65 km/hour.

- Possible damage to electrical components and function of cardiac pacemakers or cochlear implants.

- Displacement of metal objects such as metal surgical clips (aneurysm clips) and metallic objects in eyes (pre-MRI orbits).

- Local heating of tissue.

- Claustrophobia.

- Hearing impairment.

Advantages of MRI:

- Excellent contrast resolution.

- Superior soft tissue imaging due to interactions with plentiful hydrogen atoms.

- No ionizing radiation as only RF waves are used.

- Multi-planar imaging capabilities (images in every imaginable plane can be reconstructed).