MRAD 302- Specialized Imaging 2

Nuclear Medicine and PET Worksheet

A group of radioactive drugs used in diagnosis and treatment of disease id termed ______________________.  Nuclear medicine examines physiological functions of an organ on the ______________________ level. The radioactive materials are introduced into the body by the three following routes: _____________________, _______________________, & ______________________.  The most common nuclide used in nuclear medicine procedures is ___________________________.This nuclide has a physical half-life of ___________________ .

SPECT is the abbreviation for ______________________________________________________.

The typical dose for most diagnostic nuclear medicine procedures ranges between ____________________________ to ____________________________.  A bone scan can detect a fracture up to ___________________________________ post injury.

Match each of the following nuclear medicine terms to its correct definition. (Use each choice only once):

_____ A. Synonym for product decay			1. Becquerel

_____ B. Time required for disintegration 		2. Radionuclide
	of half of the original activity of a nuclide

_____C. Type of atom whose nucleus		             3.Daughter
	disintegrates spontaneously

______ D. External indication of a device 		4. Curie
	designed to enumerate ionizing events

______ E. SI unit of radioactivity			5. Equilibrium

______F. Stage in a reaction in which the		6. Half-life
	concentration of the reactive species is
              no longer changing

____ G .Traditional or standard unit of radioactivity	7. Disintegration

____H. Spontaneous nuclear transmutation		8. Count
	characterized by emission of energy or
	mass from the nucleus

Technetium 99 m has an energy level of ____________ and will allow target tissue to return to background radiation levels within: ________ hours.

PET is the acronym for ____________________________.  It is a process that demonstrates ______________________________.

PET uses radioactive compounds that emit _______________________ during the radioactive decay process.  The disappearance of the electron-positron pair and in their place two 511 keV photons that travel out in opposite directions is termed ___________________. 

 The PET scanner detector array measures _______________________.  The energy detection process used with PET is termed _____________________ imaging. PET uses radioactive compounds which include oxygen, nitrogen, _________________ and _________________.

PET radioactive compounds measure all of the following vital cellular biochemical processes: ____________________, _______________________ & ____________________________.

Most PET tracers have a half-life of _________________________.  PET can be used to investigate the location of seizure sites in patients with epilepsy who are not responding to drug therapy.  PET also plays an important role in the initial diagnosis and staging of malignancy. PET/CT scanner can determine the location of an atherosclerotic lesion and its impact on cardiac perfusion.

PET brain mapping is performed to _____________________________________________.

PET can be combined with an _____________ study to determine if any epileptic activity is present.  It is possible to acquire functional PET and anatomic CT images in the same scanning session when the correct technology is employed.