# Activity: CT Scan Parameters

## Purpose:

Outcome 3: Analyze components of image formation and appearances.

## Instructions:

- Use the chart to match the factors to their appropriate category.

- Answer the short answer questions.

| Contrast Resolution | Spatial Resolution | Both |
| --- | --- | --- |
| b) mAs | c) Motion | a) Slice thickness |
| g) Windowing | d) Pitch | e) Pixel size |
| i) Body Habitus | f) Matrix size | k) Reconstruction algorithm |
|  | h) DFOV |  |
|  | j) Focal Spot Size |  |

Factors:

- Slice thickness

- mAs

- Motion

- Pitch

- Pixel size

- Matrix size

- Windowing

- DFOV

- Body Habitus

- Focal Spot Size

- Reconstruction Algorithm

Questions

- Which of the above factors are considered “uncontrolled”?

- Motion

- Matrix size

- Body Habitus (aka. pt. size)

- Which factor listed above, is the best option to manipulate, for improving the visualization of a soft-tissue tumor found inside a patients right kidney?

Windowing (*see Romans 2nd Ed. Pg. 72 & Fig. 6-13)

- How should that factor be adjusted to improve visualization of the patient’s tumor?

↓ WW (*see Romans 2nd Ed. Pg. 72 & Fig. 6-13)

## Instructions:

- Answer the true or false questions.

Questions

- The size of a scanned object, affects spatial resolution.

*Spatial resolution determines the detectable size of an object; the size of an object has no effect on a systems spatial resolution capabilities

*Spatial resolution determines the detectable size of an object; the size of an object has no effect on a systems spatial resolution capabilities

True

  - False

- Smaller objects have higher spatial frequencies.

  - True – smaller objects can appear more often in a given space

  - False

- Contrast resolution is only affected by underexposures.

  - True – because of the uncoupling effect (aka. quantum mottle is present

  - False

- Increasing WW, decreases low-contrast detectability.

  - True – See Romans 2nd Ed. Pg. 72 & Fig. 6-13

  - False

- SNR affects both contrast and spatial resolution.

*SNR only affects contrast resolution – see Romans 2nd Ed. Pg. 71

*SNR only affects contrast resolution – see Romans 2nd Ed. Pg. 71

True

  - False

*P has a greater impact on the effective slice thickness in SDCT systems. – See Romans 2nd Ed. Pg. 70

*P has a greater impact on the effective slice thickness in SDCT systems. – See Romans 2nd Ed. Pg. 70

Pitch influences MDCT systems effective slice thickness, thereby affecting spatial resolution.

  - True

  - False

- Motion creates image blurring, and reduces the contrast resolution.

*Pt. motion affects spatial resolution – See Romans 2nd Ed. Pg. 70

*Pt. motion affects spatial resolution – See Romans 2nd Ed. Pg. 70

True

  - False

- High contrast reconstruction algorithms, improve spatial resolution.

  - True – see Romans 2nd Ed. Pg. 68

  - False

*SNR has no effect on spatial resolution; slice thickness affects spatial resolution because of volume averaging (due to the voxel size). – See Romans 2nd Ed. Pg. 66

*SNR has no effect on spatial resolution; slice thickness affects spatial resolution because of volume averaging (due to the voxel size). – See Romans 2nd Ed. Pg. 66

Slice thickness affects spatial resolution because of its relationship with signal-to-noise ratio.

  - True

  - False

- Matrix size does not affect contrast resolution, as it is an inherent factor.

  - True - because matrix size cannot be changed, it does not influence image contrast

  - False

## Instructions:

- Answer the following questions regarding image quality.

Questions

- What is SNR?

Signal-to-noise ratio. The number of x-ray photons detected per pixel in CT. High signal (i.e. photon) to noise ratios are preferred.

- What is the relationship between spatial resolution and SNR?

None. Spatial resolution is unaffected by SNR. SNR affects contrast resolution

- Identify 3 factors that affect noise in CT images.

- slice thickness (thicker slices reduce image noise)

- mAs (more quantity reduces noise)

- tissue thickness and density (decreasing these factors will reduce noise)

- How does pixel size affect spatial resolution?

The smaller the pixel size, the better the spatial resolution (i.e. smaller pixels image smaller objects)

(↓ pixel size = ↑ spatial resolution)

- What is the pixel size in mm, of a 24cm DFOV, and a 512-matrix head study?

*Ensure units of measurement are consistent: 1cm = 10mm

*Ensure units of measurement are consistent: 1cm = 10mm

*24cm DFOV x 10 = 240mm DFOV

Pixel Size = 0.47mm

- What is the pixel size in mm, of a 20cm DFOV, and a 256-matrix spine study?

Pixel size = 200mm/256

Pixel size = 0.78mm

*Ensure units of measurement are consistent: 10mm = 1cm

*Ensure units of measurement are consistent: 10mm = 1cm

What is the pixel size in cm, of a 450mm DFOV, and a 512-matrix abdominal study?

* 450mm DFOV/10 = 45cm DFOV

Pixel Size = 0.09cm

- Explain which study (head, spine, or abdominal) used the highest spatial resolution?

The head study had the highest spatial resolution, because it used the smallest pixel size (0.47mm)