MRAD 204 – Radiographic Technique 1

Lab 1- Orientation to the Medical Imaging Room,

Control Panel and Accessory Equipment

### Purpose

Provide an orientation to the x-ray imaging lab.

Enable learners to prepare a room for routine imaging.

### Instructions

Working with your group or individually: ensure your try to locate and use the equipment components listed under the four primary categories.

*Ask an instructor for assistance as needed.

Note: safety locks are engaged during labs, preventing exposures to ionizing radiation

Category #1: Console (aka. Control Panel)

*Room 1 console: consists of a control panel and computer monitor* in all other rooms, the control panel is part of the computer monitor.

- Identify the location of the on/off buttons found on the control panel (*beside the computer monitor)

- Enter the username & password for the console (*as prompted by the computer monitor):

  - Username: user

  - Password: hit the enter key twice

- Add patient “Jack Skellington” D.O.B. 22/01/1999 into the console:

  - Click: enter (bottom left of screen)

  - Follow the prompts to type the patients name & date of birth (DOB)

  - Enter the accession number: 111

  - Select the body part: hand (found under upper extremities)

  - Click: add

  - Enter a new accession number: 222

  - Select another body part: wrist

  - Click: confirm

  - Click: cancel

  - Highlight the patients name & click: start exam (top left of screen)

  - Add your initials to the “operator box” (bottom left) & hit enter

  - *to enter more patients, click: patient list (bottom right) to move back to the “patient screen”

- Locate the 3 different image receptor (IR) selectors

(*Only Room 1 has selectors on the computer monitor & control panel)

- Locate the exposure button and:

  - Press and hold the button all the way down until you hear an audible “beep”

    - Note: this is the most common way to take an exposure

    - Caution: Do not release the button to early!

  - Press and hold the button half way down (wait until you hear an audible noise and see the exposure button turn a different color) then depress the button completely

    - Note: this is called the “rotor” setting; it is reserved for patients who cannot hold still or follow breathing instructions; it decreases the life expectancy of the tube

Category #2: Accessory Equipment

- Locate the CR cassette IR’s (image receptors)

  - Identify the 3 different sizes

  - Note how to store CR’s properly (barcode away from bottom surface, black surface covered)

  - Make note of CR Reader location (*note: the CR Reader will be used in APPH 226 labs)

- Locate the gloves, cleaner, paper towel, and hand sanitizer

- Locate the shielding found in your room

  - Lead aprons, blockers, and rolling shield

- Find the lead markers

- Locate the sponges, sandbags, and pillow cases

Category #3: X-Ray Tube (aka. camera)

- Identify the different tube movements, by pressing and holding the following buttons:

*note: when moving the tube, you must press and HOLD a button until the tube locks itself into a position; only release the button early if you do not need the tube to lock itself into position.

Hint: the rails on the ceiling are also color-coded

Hint: the rails on the ceiling are also color-coded

Yellow lock

*note: Light above this button will almost never glow

  - Blue lock

  - Green lock

  - Black lock

*note: Light above this button will glow when the tube is set to 0o or 90o

  - Purple lock

  - All locks

- Move the tube into a detent (aka. lock)

*note: when the tube locks itself into a detent, you can release the button, re-press and hold the button,, then move the tube out of the detent as needed.

- Turn the collimator light on

  - Find the digital display of the field size (aka. size of light you see)

  - Adjust the field size in both directions

  - Rotate/twist the collimator head to move the field size orientation

- Find the manual measuring tape

- Activate the laser light

  - Hide the laser light using the sleeve on the tube handle

- Angle the tube 15 degrees towards the head end of the table

Category #4: Table

- Have a classmate remove their shoes and lie on the tabletop, then move the table:

  - Up & Down

  - Free float (i.e. in a circular motion)

- Locate the button that will prevent all table movement (it glows when it’s been engaged)

*only complete these activities if you have time after locating the equipment from all categories

Activity #1

- Grab a cassette of your choosing

- Place the cassette anywhere on the tabletop, and in any orientation (as long as the active surface is face up)

  - Note: the active surface of the CR cassette is the black surface

  - Do not move the cassette once it is placed on the table

- On the tube:

  - Press the “free cassette” IR button

  - Set a 0-degree tube angle

  - Use “all locks” to move the tube over the cassette

  - Use the manual measuring tape to set a 100cm SID (i.e. distance from the tube to the cassette)

  - Adjust the light field to match the cassette size

    - You may rotate/twist the collimator head as necessary to ensure the light field orientation matches the cassette orientation, as needed

Activity #2

- Move the tube anywhere over the tabletop

  - Set a 15-degree tube angle (towards the foot end of the table)

  - Use the manual measuring tape to set a 100cm SID (measure distance of tube to tabletop)

  - Adjust the light field to 10x10cm

- Grab a different size cassette

- Place the cassette closer to the middle of the tabletop, in any orientation (as long as the active surface is face up)

  - Do not place the cassette under the tube

  - Do not move the cassette once it is placed on the table

- Use the table to center the light field (from the tube) to the middle of the cassette

  - Note: you may rotate/twist the collimator head to ensure the light field orientation matches the cassette orientation, as needed