# APPH 226 – Apparatus and Image Management

# Lab 12 - CR Reader Workstations

# Detector and Extremity Phantom

### Purpose: Objective: 6.3 Compare (CR) processing to (DR) processing.

### Recognize how to operate a CR Reader.

### Instructions for Exposures: take all 3 exposures first, before processing.

Exposure #1

- Enter patient information into the console.

- Select an extremity phantom, and select the appropriate exam.

- For IR selection, use free cassette.

- Move the detector to ensure it will not be exposed.

- Position the part, use a 100cm SID and collimate to the anatomy.

- Place the marker just outside of the collimated light field.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

Exposure #2

- Add a new examination for a different phantom, and switch cassettes.

- IR selection should be set to free cassette, align the tube and new cassette using a 100cm SID.

- Position and collimate appropriately.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

mAs used:_________  EI:___________

Exposure #3

- Maintain the same setup as Exposure #2, just switch cassettes.

- Collimation and position of phantom should be unchanged.

- Double the mAs used in Exposure #2.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

mAs used:_________  EI:___________

### Instructions for CR Reader Processing

- Process Exposure 1 with the CR Reader by following these steps:

  - If the display screen is on the ‘Log-on’ screen, select: “Log-on Tech” button at bottom left

  - Select: “Study Data”

  - Select the “New Patient” icon

  - Enter the information requested by the (blue/yellow) colored boxes

  - Select: “Add a View” and choose the appropriate view for the image taken

  - Using the barcode reader, scan the cassette

  - Slide the cassette into the reader **yellow tab towards you, barcode down, black face of cassette facing the right** in a smooth motion, slide the cassette into the reader until it reaches the back.

  - Explore the tabs for post-processing functions

  - When completed, select “reject” choose a rationale and select “end study”

- Process Exposure 2 with the CR Reader by following these steps:

  - From the ‘Main Menu’ page:

  - Select: “Study Data”

  - Press the “Trauma” icon and enter the required information.

  - Reassign the procedure to an appropriate body part

- Primary Category

- Secondary Category

- View

  - Scan the cassette

  - Slide the cassette into the reader **yellow tab towards you, barcode down**

  - Explore the tabs for post-processing

  - When completed, select “reject” choose a rationale and select “end study”

- Process Exposure 3 with the CR Reader by following these steps:

  - Do not scan the barcode of the cassette

  - Slide the cassette into the reader **yellow tab towards you, barcode down**

### Recovery of an unassigned image

- From the ‘Main Menu’ page:

- Select: “Image Review”

- Press the “Unassigned Images” tab

- Use the Date, Time, & Cassette ID number to correctly identify the missing image

- Select the correctly identified ‘unassigned image’

- Select: “Assign Image”

- Type in the patient data and select “Find Study/Local Find”

- Select the correct patient data and press “Assign”

Questions:

- Which function enables users to follow correct hanging protocols? Are there any limitations to this function?

- Does the annotation function enable the user to add information other than the right and left marker? If yes, explain.

- What is the benefit of the Edit Mask/Show Mask function?

- When the masking function is used and the re-processing button is selected, is the image display affected?

- How does Windowing affect the Histogram?

- How would you define the classification of a primary, and secondary category?

- Does changing the classification of the image affect the image display? Explain.

- What is the relationship between exposure and the EI number?

- Utilizing the relationship in Question 8, what will the new EI be if the original EI was 800 with an exposure of 70 kVp and 5 mAs, and the new exposure is 70 kVp and 10 mAs?