# Worksheet – Grids- ANSWER KEY

- Define:

  - Grid ratio: the height of the grid divided by the interspace width

  - Grid frequency: the number of grid strips per cm/inch

  - Contrast Improvement Factor (k): the ability of the grid to improve contrast

  - Bucky Factor: the amount that the radiographic technique must be increased to produce the same radiographic density/image receptor signal

| Grid | Description | Advantages | Disadvantages |
| --- | --- | --- | --- |
| Parallel | -simplest type -all lead strips running parallel | - easiest to manufacture - only one type of grid error possible | -grid cut off towards edges, especially with short SID |
| Crossed | -lead strips running parallel to long and short axis of the grid | -more efficient at cleaning up scatter | -positioning is critical -cannot angle the x-ray beam -increase in exposure is substantial |
| Focused | -designed to minimize grid cut off -lead strips coincide with the divergence of the beam | -when properly positioned, no grid cut off -works with tube angles | -more difficult to manufacture - care needs to be taken when positioning or grid errors will result |

2.

- A technologist is taking a mobile abdomen on a patient in ICU using 80 kVp @ 12 mAs and a 8:1 grid. What will the exposure factors be if the technologist was using a:

  - 5:1 grid

mAs

GCF

12 mAs	4

1

mAs

2

=	1

GCF

2

mAs	=	2

2

mAs

= 6	80 kVp 6 mAs

2

  - 12:1 grid

mAs

GCF

12 mAs	4

1

mAs

2

=	1

GCF

2

mAs	=	5

2

mAs

= 15	80 kVp 15 mAs

2

  - No grid

mAs

GCF

12 mAs	4

1

mAs

2

=	1

GCF

2

mAs	=	1

2

mAs

= 3	80 kVp 3 mAs

2

- Exposure 1 is taken with a 12:1 grid using 85 kVp 20 mAs. What will the new exposure factors be if Exposure 2 is taken with an 8:1 grid?

mAs

1

mAs

2

GCF

=	1

GCF

2

20 mAs	5

mAs	=	4

2

mAs

= 16	85 kVp 16 mAs

2

- Exposure 1 is taken with a 5:1 grid using 65 kVp 10 mAs. What will the new exposure factors be if Exposure 2 is taken using an 8:1 grid?

mAs

1

mAs

2

GCF

=	1

GCF

2

10 mAs	2

mAs	=	4

2

mAs

= 20	65 kVp 20 mAs

2

- Exposure 1 is taken with an 8:1 grid using 75 kVp 32 mAs. What will the new exposure factors be if Exposure 2 is taken with no grid?

mAs

1

mAs

2

GCF

=	1

GCF

2

32 mAs	4

mAs	=	1

2

mAs

= 8	75 kVp 8 mAs

2

- An exposure is taken with a 16:1 grid using 105 kVp 12 mAs. A second exposure is to be taken with an 8:1 grid. What will the new exposure factors be to double the radiographic intensity for the second exposure?

mAs

1

mAs

2

GCF

=	1

GCF

2

12 mAs	6

mAs	=	4

2

mAs

= 8 x2 = 16	105 kVp 16 mAs

2

- Exposure 1 is taken with a 12:1 grid using 80 kVp 8 mAs. What will the new exposure factors be if Exposure 2 is taken with a 16:1 grid?

mAs

1

mAs

2

GCF

=	1

GCF

2

8 mAs	5

mAs	=	6

2

mAs

= 9.6	80 kVp 9.6 mAs

2

- Exposure 2 is taken with a 12:1 grid using 85 kVp 20 mAs. What were the original exposure factors be if Exposure 1 was taken with a 16:1 grid?

mAs

1

mAs

2

GCF

=	1

GCF

2

mAs 1	6

20	=	5

mAs

= 24	85 kVp 24 mAs

1