APPH 226 – Apparatus and Image Management

# Lab 12 - Exposure Indicator (EI) – Phantom

### Purpose: Identify how to assess the quality of a digital image.

Objectives: 6.4 Evaluate digital image characteristics

1.6 Monitor an exposure for technical errors

### Instructions

Exposure #1

- Enter the patient information into the console.

- Select the appropriate exam for the phantom and set up the detector.

- Use a manual technique of 75kVp @ 0.5mAs.

- Position the phantom with minimal OID.

- Align the tube and detector, use an SID of 110cm. Collimate and center to the phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor before exposing.

Record the EI number readout:  ________

Exposure #2

- Maintain the same room setup as Exposure #1.

- Wait approximately 1 minute before taking the next exposure.

- Set a manual technique of 75kVp @ 1.6mAs

- Ensure everyone remains outside the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor before exposing.

Record the EI number readout: _________

Exposure #3

- Maintain the same room setup as Exposure #1.

- Wait approximately 1 minute before taking the next exposure.

- Set a manual technique of 75kVp @ 6.3mAs

- Ensure everyone remains outside the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor before exposing.

Record the EI number readout: _________

Exposure #4

- Maintain the same room setup as Exposure #1

- Wait approximately 1 minute before taking the next exposure.

- Set a manual technique of 75kVp @ 12.5mAs

- Ensure everyone remains outside the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor before exposing.

Record the EI number readout: __________

### Discussion:

- By observation only, is it possible to visually determine which exposures were over and/or underexposed?

- Why is it important for the technologist to evaluate the EI number?

- What does the EI number represent/where is it measured?

- Which of the exposures could be considered diagnostic, and why?

- Why is the EI readout important to monitor when exposing with AEC?