Purpose

Lab 11

- Plan and perform an imaging procedure for the ankle, foot and calcaneus.

Instructions

  - As a group decide who will begin performing the following roles:

    - Technologist

    - Patient

    - *Observing technologist (applies to groups of 3)

  - The acting technologist should begin positioning a patient for Routine Ankle imaging

    - Complete all 3 projections

    - Maintain a workflow (i.e. select pt. info on the console, prep the exam room etc.)

      - Audibly say “beep” when a pt. is properly positioned and you are ready to “expose”

  - Switch roles and repeat step 2 (until everyone has completed positioning for Routine Ankle projections)

  - Repeat steps 2-3, except practice positioning a patient for Routine Calcaneus imaging

  - Repeat steps 2-3, except practice positioning a patient for Routine Foot imaging

  - As time permits, repeat steps 2-5 to successfully accomplish the lab goals

Goals:

- Perform an entire Routine Ankle exam within 6 minutes or less

- Perform an entire Routine Calcaneus exam within 4 minutes or less

- Perform an entire Routine Foot exam within 6 minutes or less

- Practice both anatomical sides (as time permits)

*Overall goal: perform room setup within 2 minutes (includes pt. double id.), and 2 minutes positioning per projection*

Reminders:

- Ensure the IR selector is set appropriately to “cassette” *even if using the “wireless IR”

- Lock must be “on” for all straight ray projections

o Check SID manually (and adjust when changing CR angles)

- Ensure required anatomy is centered to the IR

- Follow Edenbrook’s Marker Rules

- Practice both anatomical sides (aka. right & left ankle, calcaneus, and foot routines)

- Re-set the room after each exam to practice managing the equipment

- Ask your supervising instructor for assistance as needed