APPH 226 – Apparatus and Image Management

Creating an Exposure

Purpose

Objective: 1.1 Describe the sequence of events while creating an image.

Become familiar with the equipment and process required to obtain a radiographic image on a digital radiographic unit.

Instructions

Turning the System “On”:

- Locate the small blue binder labelled “Tech Tips” that corresponds to the assigned room.

- Find the page that lists the instructions for turning the system “on”.

- Turn the equipment on, as per the instructions.

- Do NOT move the detector while waiting for the equipment to turn on.

Exposure #1

Once the equipment is ready:

- Enter patient information into the console (ensure it is workplace appropriate).

- Select the appropriate procedure for the phantom provided.

- Use the recommended manual technique (APR).

- Place the smaller phantom on the table.

- Center the tube to the detector.

- Check that the distance is 110cm.

- Use the table to move the phantom into the center of the light.

- Collimate to have ½’’ of light around the phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

Record the values for the following important components that affect image quality:

  - What was the mAs readout? _____________

  - What was the EI/EI_s number readout? ___________

  - What was the kVp readout? __________

Exposure #2

- Place the larger phantom in front of the upright wall detector.

- On the computer, select the proper procedure.

- Ensure the appropriate detector is selected.

- Use AEC as the technique.

- Center the tube to the detector.

- Check that the distance is 110cm.

- Open the collimation field as much as possible.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

Record the values for the following important components that affect image quality:

  - What was the mAs readout? _____________

  - What was the EI number readout? __________

  - What was the kVp readout? __________

  - Which AEC cells were activated? __________

Turning the System “Off”:

- Locate the small blue binder labelled “Tech Tips” that corresponds to your assigned room.

- Find the page that lists the instructions for turning the system “off”.

- Shut down the equipment, as per the instructions.