# Activity - CT Artifacts

## Purpose:

Objective 4.6: Identify common CT image artifacts.   Instructions:   Refer to Romans 2nd Ed. pg. 78-83 to complete the chart.

| Type of Artifact | Cause | Appearance | How to: Avoid/Fix |
| --- | --- | --- | --- |
| Beam Hardening | - Attenuation |  |  |
| Partial Volume | - More than 1 type of tissue in a voxel |  |  |
| Aliasing | - Under sampling |  |  |
| Edge Gradient Effect | - Irregularly shaped objects with large density differences (ex. Barium next to air in the stomach) |  |  |
| Motion | - The patient - Voluntary & Involuntary |  |  |
| Metallic | - Density of metal exceeds HU value range of the system (aka. dynamic range) |  |  |
| Out-of-Field | - Anatomy that is outside of the SFOV |  |  |
| Rings | - Malfunctioning or mis calibrated detector elements (aka. imperfect detector elements) |  |  |
| Tube Arcing | - Large electrical potential differences |  |  |
| Helical Acquisition | - Motion of the beam - Data interpolation - Example = Windmill artifact |  |  |
| Cone Beam | - Helical MDCT scans (more pronounced in outer detector rows) - Data Interpolation - Cone Beam Geometry |  |  |
| Noise | - Quantum Noise - Inherent Physical Limitations - Reconstruction parameters |  |  |