# APPH 226 – Apparatus and Image Management

# Lab 6 - Grids & Exposure Technique, knee phantom

### Purpose

Objective: 5.3 Evaluate grids and their effect on image quality.

Correlate how grids influence technical factor selection.

### Instructions

Exposure #1

- Enter the patient information into the console, select knee.

- Select table detector.

- Instructor will remove the grid.

- Turn AEC off and set 70kVp @ 15mAs.

- Center the tube and detector using an SID of 110cm.

- Collimate to a 20 x 25cm field size.

- Position the knee body part phantom and as close to the detector as possible.

- Center over the midline of the body part phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Exposure #2

- Maintain the same setup as Exposure #1.

- Move the knee body part phantom about 15-20 cm away from the detector (OID).

- Center to match the centering used in Exposure #1.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Image Evaluation:

- Describe any differences between the 2 images.

- What type of technique was simulated in Exposure #2?

- Which exposure produced more radiographic density.

Exposure #3

Grid Ratio: 12:1 grid

- Select table detector.

- Turn AEC off and set 70kVp @ 15mAs.

- Center the tube and detector using an SID of 110cm.

- Instructor will place the grid in slot above the detector.

- Collimate to a 20 x 25cm field size, center over the midline of the knee body part phantom.

- Ensure everyone is out of the room and the door is closed.

- Obtain verbal confirmation with direct supervision from an instructor.

- Take the exposure.

- Record the EI readout: _________.

Grid ratio, type, air gap (OID) and review.