APPH 226 - Worksheet - Size Distortion Formulas and Calculations – ASWER KEY

Magnification or Size Distortion can be assessed by calculating the magnification factor.

MF =   SID   	  ***  SOD = SID – OID

SOD

Example:

If the SID is 40 inches, and the SOD is 30 inches, what is the magnification factor?

40 inches    = 1.33

30 inches

The magnification factor is 1.33

The magnification will be 33% or the image will be 133% of the object size.

- What is the magnification factor if the SID is 40 inches and the OID is 6 inches? SOD = 40-6 = 34                 MF = 40/34 = 1.18

- What is the magnification factor if the SOD is 72 cm and the OID is 6 cm?

SOD + OID = SID = 72 + 6 = 78cm MF = 78/72 = 1.08

- What is the magnification factor if the SID is 72 inches and the SOD is 68 inches? MF = 72/68 = 1.06

- What is the magnification factor is the OID is 5 cm and the SID is 115 cm?

SOD = 115- 5 = 110 cm MF = 115/110 = 1.05

- What is the magnification factor if the SOD is 48 inches and the OID is 2 inches? SOD + OID = SID = 48 + 2 = 50             MF = 50/48 = 1.04

- What is the magnification factor if the SOD is 85 cm and the SID is 100 cm?

MF = 100/85 = 1.18

The magnification factor permits calculation of the actual size of an image using this formula:

Object size =     	  Image

Magnification Factor

Example:

If an object is 2 inches in diameter on the image, and is measured to be 3 inches away from the IR, what is its actual size if the SID used is 40 inches?

This is the information given:    Image size = 2 inches     SID = 40 inches        OID = 3 inches Step 1.  Solve for the magnification factor first.

M = SID  	  =    40    = 1.08

SOD   	 37 Step 2.  Solve for the object size

O =   2 inches      = 1.85 inches

1.08 	The object size is 1.85 inches in diameter.

  - If the image size is 24 inches, and there is a magnification factor of 2, what is the size of the object that was x-rayed? O  = 24/2 = 12 inches

  - If a phalange having an area of 2 inches is x-rayed with a 4 inch OID using a 40 inch SID, what would be the size of the phalange on the image?

SOD = SID – OID = 40-4 = 36    MF = SID/SOD = 40/36 = 1.11      I= (O)(MF)=2x1.11 = 2.22 inches

  - If the object is 13 cm and the image size is 15cm, what is the magnification factor?

MF=I/O = 15cm/13cm = 1.15

  - If the SID is 50 inches, the object is 2 inches and the OID is 2 inches, what is the image size?

SOD = SID – OID = 50-2=48   MF=SID/SOD = 50/48 = 1.04    I=(O)(MF)= 2x 1.04 = 2.08 inches

Percent of Magnification can be found when we know the image and object size. % Magnification of Object =    I-O      x 100

O

Example:

The image of a calcaneus measures 6 cm across, and the actual size of the calcaneus is 5 cm.

What is the percent magnification of the object?

6cm-5cm   x 100 =  20 % magnification of the object

5cm

1.	What is the percent magnification if the object is 11 inches long, and the image is 14 inches long?

(14-11)/11=0.27 x 100 = 27.3%

  - If the object measures 12 inches and shows up on the image as 24 inches, what is the magnification factor? MF = I/O = 24/12 = 2

  - What is the percent of magnification in Question 2 (above question)? (24-12)/12=1 x 100 = 100%

# Practice Questions

- Image size = 13 cm   Magnification factor (MF) = 1.7 Calculate the object size.

O=I/MF = 13/1.7 = 7.6 cm

- Image size = 8 inches    Object size = 5 inches

Calculate the magnification factor

MF = I/O = 8/5 = 1.6

- SID = 40 inches     Image size = 9 inches    Object size = 7.5 inches Calculate the SOD

MF = I/O = 9/7.5 = 1.2      SOD = SID/MF = 40/1.2 = 33 inches

- Object size = 13 cm      Magnification Factor (MF)= 1.19

Calculate the image size

I=(MF)(O) = (1.19)(13) = 15.5 cm

- A mass measures 1.2 cm on an image.  The OID is 8 cm, and the SID is 110 cm.  What is the actual size of the mass?

SOD = SID – OID = 110-8 = 102

MF = 110/102 = 1.08

O = I/MF = 1.2cm/1.08 = 1.1 cm

- SID = 50 inches         Object size = 3 inches   OID = 4 inches Calculate the image size.

SOD = SID – OID = 50-4 = 46        MF = SID/SOD  = 50/46 =1.1 I = (MF)(O) = 1.1 x 3 = 3.3 inches

- Image size= 26 cm  Object size = 21 cm Calculate the percent magnification

(I-O)/O   x 100 = (26-21)/21 = 23.8 or 24%

- Image size = 16 cm      Magnification Factor (MF) = 1.8

Calculate the percent magnification

O = I/MF = 16/1.8 = 8.9 	 (16-8.9)/8.9= 0.80 x 100 = 80%

- Image size = 26 cm   Magnification factor (MF) = 1.3

Calculate the object size.

O=I/MF = 26/1.3 = 20 cm

- A gallbladder stone measures 2 cm on the radiograph.  The SID is 100cm and the SOD is 90 cm.  What is the actual size of the stone?

MF = SID/SOD= 100/90 = 1.11     O=I/MF = 2/1.11 = 1.8 cm

APPH 226